# Internet of Things: Architectures, Protocols and Standards

## Comprehensive Technical Summary

Authors: Simone Cirani, Gianluigi Ferrari, Marco Picone, Luca Veltri Publisher: John Wiley & Sons, 2019 (First Edition) ISBN: 9781119359678

## Preface and Scope

The book presents a research-grounded perspective on the IoT ecosystem, developed primarily between 2012–2015 at the University of Parma, largely within the EU FP7 CALIPSO project (Connect All IP-based Smart Objects!, 2012–2014). It addresses heterogeneous communication technologies through four unifying concepts: interoperability, discoverability, security/privacy, and cloud/fog computing. The intended audience is academic and industrial professionals with solid networking background. The book explicitly positions its protocols and architectures as representative, not universal.

## Chapter 1 — Preliminaries, Motivation, and Related Work

### 1.1 What Is the Internet of Things?

- Definition: The IoT encapsulates a vision in which billions of objects with embedded intelligence, communication means, and sensing/actuation capabilities connect over IP networks.

- Core concept: Smart objects and cyber-physical systems — referred to as "things" — are physical objects augmented with:

  - Micro-controllers

  - Optical and/or radio transceivers

  - Sensors and actuators

  - Protocol stacks suitable for constrained environments

- Device constraints: Limited memory, limited available energy, stringent low-cost requirements.

- Internet evolution: The Internet has transitioned from hardware-driven (computers, Ethernet cables) to market-driven (cloud services, platforms), necessitating open environments and interoperable architectures.

- Edge Computing paradigm: Introducing computing resources at the edge of access networks yields:

  - Low latency

  - Real-time capabilities

  - Context-awareness

  - Edge nodes act as interfaces to data streams from connected devices

- Big Data processing: Stored Big Data can be processed via machine learning and deep learning to transform raw sensor data into actionable information.

### 1.2 Wireless Ad-hoc and Sensor Networks: The Ancestors without IP

- Wireless Sensor Networks (WSNs): Emerged in the early 2000s as a class of networks promising wide-scale interaction between people and surroundings. Three core attributes:

  - Wireless: Eliminates wiring, supports mobility and ease of deployment

  - Sensor: Provides perception and interaction with the physical environment

  - Networks: Enables systems built from distributed, communicating devices

- Communication paradigm shift: WSNs replaced the traditional address-centric approach (explicit source/destination in every packet) with a data-centric approach — "address blindness" — enabling data diffusion strategies without explicit routing to named endpoints.

- Network topology: Sources placed in monitored areas; sinks located at accessible points with adequate storage. Data scheduling policies:

  - Time-driven (periodic)

  - Event-driven

  - Query-driven (response to requests)

  - Hybrid combinations

- Ad-hoc networks: Infrastructure-less, cooperation-based, opportunistic networks. Key research problems addressed:

  - Self-configuration and self-organization

  - Cooperative operations among heterogeneous members

  - Multi-hop peer-to-peer routing

  - Network self-healing and robustness

  - Seamless mobility management and dynamic topology support

### 1.3 IoT-Enabled Applications

#### 1.3.1 Home and Building Automation

- Growing smart home market; fragmented vertical market segments.

- Applications: home security, energy efficiency, healthcare (remote monitoring for elderly and disabled), air-quality monitoring.

- The IoT is expected to create a new level of interoperability between commercial home automation solutions, moving from luxury/security toward broader connected solutions.

#### 1.3.2 Smart Cities

- Cities as complex ecosystems requiring efficient monitoring.

- Domains: healthcare, media, energy, environment, safety, public services.

- IoT technologies collect city-state data and disseminate it to citizens and public authorities.

#### 1.3.3 Smart Grids

- Definition: An electrical grid integrating smart meters, smart appliances, renewable energy resources, and energy-efficient systems.

- Power Line Communications (PLC): Uses existing electrical cables to transmit data.

  - EU standards permit low-bit-rate data (< 50 Kbps) in the 3–148 kHz frequency band.

  - Enables smart metering, energy consumption reporting, remote control in industrial automation.

- Limitation: Power lines are imperfect channels due to variations in location, time, frequency band, and connected equipment.

- IoT + PLC jointly improve robustness; resource-constrained IoT devices with internet-standard solutions (IETF, ETSI, W3C) are proposed as key components of future smart grids.

#### 1.3.4 Industrial IoT (IIoT)

- Definition: IoT applied in manufacturing, logistics, oil/gas, transportation, energy/utilities, mining, aviation, and related industries.

- Real-time requirements: Industrial real-time = sub-millisecond (vs. consumer Internet ≈ seconds).

- Reliability: "Best effort" Internet is insufficient; power grid or factory failures carry far graver consequences.

- Key IIoT capabilities:

  - Sensor-driven computing: Converting sensed data into insights

  - Industrial analytics: Turning sensor/source data into actionable insights

  - Intelligent machine applications: Integrating sensing and intelligence into machinery

- Initiatives: GE ($1B+ incremental revenue from IIoT in 2014), Germany's Industrie 4.0, Netherlands' Smart Factory, France's Industry of the Future, China's "Made in China 2025."

- Standardization is identified as a critical bottleneck due to multiple incompatible factory-floor protocols.

#### 1.3.5 Smart Farming

- Challenges: Population growth, urbanization, environmental degradation, climate change, changing food preferences.

- Precision agriculture: Uses drones, sensor networks, and cloud platforms for real-time granular data collection.

- Applications: Fleet management, livestock monitoring, fish farming, forest care, indoor city farming.

- Wireless communication categories:

  - Short-range: IEEE 802.11 (Wi-Fi), IEEE 802.15.1 (Bluetooth), IEEE 802.15.4 (ZigBee/6LoWPAN) — ISM band at 2.400–2.4835 GHz.

  - Long-range: LoRA in the 868–870 MHz band — trades lower data rates (hundreds of kbit/s) for longer range.

- Application classification: Environmental monitoring, precision agriculture, M2M communications, facility automation, traceability systems.

## Chapter 2 — Standards

### 2.1 "Traditional" Internet Review

#### 2.1.1 Physical/Link Layer

##### IEEE 802.3 (Ethernet)

- Definition: IEEE standard for wired Ethernet LAN technology, defining Physical Layer and MAC sublayer of the Data Link Layer.

- Evolution:

  - 1982: Ethernet II — 10 Mbit/s over thick coaxial cable with type field

  - 1983: 10BASE5 ("thicknet") — type field replaced by length field

  - 1985: 10BASE2 ("thinnet"/"cheapnet") — thinner coaxial cable

  - 802.3i: 10BASE-T over twisted pair

  - 802.3j: 10BASE-F over fiber optic

  - 802.3bz (2016): 2.5GBASE-T and 5GBASE-T over Cat-5/Cat-6 twisted pair

- MAC mechanism — CSMA/CD (Carrier Sense Multiple Access with Collision Detection):

  - Station senses medium; defers if busy

  - If medium is free, station transmits

  - If collision detected: transmission stops, jam signal sent, random backoff before retry

  - CSMA/CD is not usable in wireless networks (cannot detect collisions without full-duplex radio)

##### IEEE 802.11 (Wi-Fi)

- Definition: Set of PHY/MAC specifications for Wireless LANs (WLANs) in 900 MHz, 2.4, 3.6, 5, and 60 GHz bands.

- Architecture: Cellular structure:

  - Cell / Basic Service Set (BSS): Controlled by an Access Point (AP)

  - Distribution System (DS): Backbone connecting APs (usually Ethernet)

  - Extended Service Set (ESS): Entire interconnected WLAN

- Notable amendments:

  - 802.11ac (2013): Very high throughput below 6 GHz; multi-user MIMO, wider channels

  - 802.11ah (2016): Sub-GHz license-exempt operations (sensor networks, smart metering)

  - 802.11ai: Fast initial link setup

- MAC mechanism — CSMA/CA (Collision Avoidance):

  - CD cannot be used in wireless (requires full-duplex radio; not all stations hear each other)

  - Procedure:

    - Station senses medium; defers if busy; transmits if free for the Distributed Interframe Space (DIFS)

    - Receiver checks CRC and sends ACK; absence of ACK triggers retransmission

  - Virtual Carrier Sensing (RTS/CTS mechanism):

    - Transmitter sends Request To Send (RTS) — includes source, destination, transaction duration

    - Receiver responds with Clear To Send (CTS) — repeats duration

    - All stations receiving RTS or CTS set their Network Allocation Vector (NAV) for the duration, reserving the medium

    - Reduces "hidden terminal" collisions and shortens collision overhead to short RTS duration

#### 2.1.2 Network Layer

##### IPv4 vs. IPv6

- IPv4:

  - 32-bit addresses in dotted decimal notation (e.g., 192.149.252.76)

  - Total address space: 2^32 = 4,294,967,296

  - Variable-length header: 20–60 bytes (depending on options)

  - IPSec optional; options integrated into header fields

  - Address types: unicast, multicast, broadcast

  - Private ranges (RFC 1918): 10.0.0.0/8, 172.16.0.0/12, 192.168.0.0/16 (not Internet-routable)

  - Minimum MTU: 576 bytes

  - Address assignment via DHCP or static configuration

  - NAT used to extend address limitations

- IPv6:

  - 128-bit addresses in hexadecimal notation (e.g., 3FFE:F200:0234:AB00)

  - Total address space: 2^128 ≈ 3.4 × 10^38

  - Fixed-length header: 40 bytes — no IP header options

  - Options supported via extension headers: AH, ESP, hop-by-hop, routing, fragment, destination

  - IPSec support mandatory

  - No NAT by design

  - Address types: unicast, multicast, anycast (no broadcast)

  - Addresses can be public or temporary (for privacy)

  - Address lifetimes: preferred lifetime (shorter) and valid lifetime; after valid expiry, address is unusable

  - Self-assigned via Stateless Address Auto-Configuration (SLAAC) or DHCPv6

  - Minimum MTU: 1280 bytes — link-layer must fragment/defragment for links with smaller MTU (e.g., IEEE 802.15.4)

  - Key for IoT: the enormous address space enables addressing of all things on Earth for decades

#### 2.1.3 Transport Layer

##### TCP (Transmission Control Protocol)

- Connection-oriented, reliable, ordered, flow-controlled.

- Header: 20 bytes mandatory (up to 60 bytes with options).

- Key header fields:

  - Source/Destination Port (2 bytes each): communication endpoints

  - Sequence Number (4 bytes): ordering of message groups

  - Acknowledgment Number (4 bytes): confirms received segments

  - Data Offset (4 bits): total TCP header size in 4-byte multiples (5 = 20 bytes, 15 = 60 bytes)

  - Reserved (3 bits): always zero; maintains 4-byte alignment

  - Control Flags (9 bits total — 6 standard + 3 extended): manage data flow; one flag initiates connection reset

  - Window Size (2 bytes): regulates data sent before requiring ACK; too small → slow, too large → link saturation

  - Checksum (2 bytes): error detection by receiver

  - Urgent Pointer (2 bytes): marks priority data subset (used with URG flag)

  - Options (0–40 bytes): special acknowledgment and window-scaling algorithms

- Establishes connection with 3-way handshake (SYN, SYN-ACK, ACK).

##### UDP (User Datagram Protocol)

- Connectionless, best-effort, stateless.

- Header: 8 bytes total.

- Key header fields:

  - Source/Destination Port (2 bytes each)

  - Length (2 bytes): total datagram size (header + data); range 8 to ~65,000 bytes

  - Checksum (2 bytes): integrity verification

- No ordering, no tracking, no flow control.

- Preferred for IoT: stateless nature suits high-volume, low-overhead constrained communication.

#### 2.1.4 Application Layer

##### HTTP (Hypertext Transfer Protocol)

- Definition: Stateless text-based request/response protocol operating over TCP.

- Key features:

  - URLs: Address resources targeted by requests

  - Methods (Verbs): Define semantics of operations (GET, POST, PUT, DELETE, etc.)

  - Status codes: Standardized response information:

    - 1xx Informational, 2xx Success, 3xx Redirection, 4xx Client Error, 5xx Server Error

- HTTP/2 (2015): Focuses on efficient communication — latency reduction, header compression, parallel flows over single connection.

##### AMQP (Advanced Message Queue Protocol)

- Current version: 1.0 (OASIS standard)

- Purpose: Asynchronous complement to HTTP; interoperable messaging for Message-Oriented Middleware (MOM).

- Defines wire format: the byte-stream format for transmission; any AMQP client works with any MOM implementing AMQP.

- Components:

  - Queues: Message storage facilities

  - Exchange: Topic-based router for message dispatching

  - Routing key: Dot-separated topic syntax for selective message dispatch

- Not a true pub/sub protocol but enables pub/sub patterns via exchanges and queues.

##### SIP (Session Initiation Protocol)

- Application-layer signaling protocol for establishing, modifying, and terminating multimedia sessions (voice, video).

- Used as a reference for the CoSIP (Constrained SIP) protocol developed for IoT.

### 2.2 The Internet of Things Standards

#### 2.2.1 Designing the Architecture of an IP-based IoT

- Principle: Enabling all IP-based smart objects to connect using a layered approach paralleling the traditional Internet stack.

- IoT-specific challenges: Constrained devices, limited bandwidth, low power, lossy links.

#### 2.2.2 Physical/Link Layer

##### IEEE 802.15.4 and ZigBee

- IEEE 802.15.4: Low-rate wireless personal area network (LR-WPAN) standard.

  - Operates primarily in the 2.4 GHz ISM band (also 868/915 MHz bands)

  - Low power consumption; designed for battery-operated, constrained devices

- ZigBee: Built on IEEE 802.15.4 PHY/MAC, adding network and application layers.

  - Suitable for short-range, low-data-rate IoT applications (home automation, industrial monitoring)

##### Low-Power Wi-Fi

- Adaptations of IEEE 802.11 for low-power operation.

- IEEE 802.11ah (sub-GHz): Specifically designed for IoT sensor networks and smart metering; longer range than 2.4 GHz Wi-Fi.

##### Bluetooth and BLE (Bluetooth Low Energy)

- Classic Bluetooth (IEEE 802.15.1): Designed for moderate data rates over short distances.

- BLE: Optimized for low power consumption; suitable for IoT devices requiring infrequent, small data transmissions (wearables, beacons, sensors).

- BLE uses frequency hopping in the 2.4 GHz band; supports advertising and connection-based modes.

##### Power Line Communications (PLC)

- Data transmitted over existing electrical wiring.

- Useful where wireless is impractical; relevant for smart grid and industrial automation.

- Limitation: Power lines are noisy, frequency-dependent, and vary with connected equipment.

#### 2.2.3 Network Layer — 6LoWPAN Adaptation Layer

- Problem: IPv6 minimum MTU is 1280 bytes; IEEE 802.15.4 maximum frame size is 127 bytes (with ~102 bytes available for payload after MAC overhead).

- 6LoWPAN (IPv6 over Low-Power Wireless Personal Area Networks): IETF adaptation layer enabling IPv6 packets to be sent over IEEE 802.15.4 links by providing:

  - Header compression: Reduces IPv6 40-byte header significantly (often to 2–3 bytes) using shared context

  - Fragmentation and reassembly: Splits IPv6 packets into 802.15.4-sized frames

  - Mesh addressing: Supports multi-hop routing at the link layer

#### 2.2.4 Transport Layer (IoT)

- UDP is preferred over TCP for IoT due to statelessness and minimal overhead.

- DTLS (Datagram TLS): Security for UDP-based IoT communication (counterpart of TLS for TCP).

#### 2.2.5 Application Layer

##### CoAP (Constrained Application Protocol)

CoAP is the primary application-layer protocol for constrained IoT environments, defined in RFC 7252. It is designed to follow a REST architecture while being optimized for machine-to-machine (M2M) communication.

Core Design Principles:

- Runs over UDP (not TCP) to minimize overhead

- Supports unicast and multicast requests

- Very low overhead and complexity

- Asynchronous message exchange

- Interoperability with HTTP through proxies

Message Types: CoAP defines four message types, differentiated by the 2-bit Type (T) field in the header:

| Type | Code | Description |
| --- | --- | --- |
| CON (Confirmable) | 0 | Requires ACK; retransmitted until ACK or RST received |
| NON (Non-confirmable) | 1 | No ACK required; used for non-critical data |
| ACK (Acknowledgment) | 2 | Confirms receipt of a CON message |
| RST (Reset) | 3 | Indicates that a CON or NON message cannot be processed |

Message Header Structure (4 bytes fixed):

- Version (Ver): 2 bits — always 01 (version 1)

- Type (T): 2 bits — CON/NON/ACK/RST

- Token Length (TKL): 4 bits — length of the Token field (0–8 bytes)

- Code: 8 bits — request method or response code

  - Request codes: 0.01 GET, 0.02 POST, 0.03 PUT, 0.04 DELETE

  - Response codes follow HTTP analogy (e.g., 2.05 Content, 4.04 Not Found)

- Message ID: 16 bits — unique identifier for matching ACK/RST to CON/NON; detects duplicates

- Token: 0–8 bytes — correlates requests to responses; allows multiplexing

CoAP Request Methods:

- GET: Retrieve representation of a resource

- POST: Create or process a resource; result in 2.01 Created (with Location-Path) or 2.04 Changed

- PUT: Update an existing resource

- DELETE: Remove a resource

CoAP Response Codes (class.detail format):

- 2.01 Created, 2.04 Changed, 2.05 Content

- 4.00 Bad Request, 4.04 Not Found, 4.05 Method Not Allowed

- 5.00 Internal Server Error

Piggybacked vs. Separate Responses:

- Piggybacked: ACK carries the response payload immediately — 2 messages total

- Separate response: ACK sent first (empty), then CON response later — 4 messages total (useful when processing takes time)

CoAP Options: Options are encoded using Type-Length-Value (TLV) format with a delta encoding scheme:

- Each option field: Option Delta (4 bits) + Option Length (4 bits) + Option Value

- Option Number = sum of all preceding option deltas (options must appear in ascending order)

- Extended Option Delta:

  - Delta 0–12: no extension

  - Delta = 13: 1 byte extension added; actual option number = 13 + extended value (max 268)

  - Delta = 14: 2 bytes extension added; actual option number = 269 + extended value (max 65,804)

- Extended Option Length: same encoding as delta

Key CoAP Options:

| Option | Number | Purpose |
| --- | --- | --- |
| If-Match | 1 | Conditional requests |
| Uri-Host | 3 | Internet host of resource |
| ETag | 4 | Resource version validation |
| Uri-Port | 7 | Transport port of resource |
| Uri-Path | 11 | Path segments (repeatable) |
| Content-Format | 12 | Payload media type |
| Max-Age | 14 | Max cache lifetime (seconds) |
| Uri-Query | 15 | Query parameters (repeatable) |
| Observe | 6 | Resource observation subscription |
| Block1 | 27 | Block-wise upload control |
| Block2 | 23 | Block-wise download control |

Content Format Identifiers (CoAP Content Format Registry):

| Media Type | Encoding | ID |
| --- | --- | --- |
| text/plain | charset=utf-8 | 0 |
| application/link-format | — | 40 |
| application/xml | — | 41 |
| application/octet-stream | — | 42 |
| application/exi | — | 47 |
| application/json | — | 50 |

Resource Observation (RFC 7641 — Observe Option):

- Problem addressed: Polling via repeated GET requests is inefficient for battery-operated constrained devices and may miss intermediate state changes.

- Observer Pattern: Client registers with server; server sends notifications whenever resource state changes.

- Procedure:

  - Client sends GET with Observe = 0 (register) and a specific Token

  - Server replies with 2.05 Content + Observe value + current resource state

  - Server sends subsequent 2.05 notifications with the same Token and incrementing Observe values on state changes

  - Client deregisters by sending GET with same Token but Observe = 1

- Observe option values:

  - 0: Registration (start observing)

  - 1: Deregistration (stop observing)

  - Other values: Used to sort notifications and discard outdated state (since UDP may reorder)

- Server overhead: Server maintains a list of registered clients (endpoint + token) per observable resource; resources are marked observable using the obs attribute in CoRE Link Format.

Block-wise Transfers (RFC 7959 — Block1/Block2 Options):

- Problem: CoAP operates over UDP (max 65,536 bytes), but constrained devices may have small buffers (e.g., 128 bytes).

- Block1 option: Controls request body transfer (upload) in blocks

- Block2 option: Controls response body transfer (download) in blocks

- Provides minimal but sufficient mechanism to split and reassemble large payloads

##### CoSIP — Constrained SIP Protocol

- Purpose: Lightweight session initiation protocol for IoT environments, analogous to SIP but designed for constrained devices.

- Built on: CoAP as the transport mechanism

- Session phases:

  - Session Initiation: UAC (User Agent Client) sends CON POST to UAS (User Agent Server) with Observe = 0; UAS responds with 2.01 Created + Location-Path (session_id) + session description; CoAP ACKs exchanged for reliability (4-message exchange minimum)

  - Session Tear-down: UAC or UAS initiates graceful termination; frees allocated processing/memory resources

  - Session Modification: Renegotiation of session parameters without full re-establishment

### 2.3 The Industrial IoT

- IIoT requires deterministic, real-time (sub-millisecond) communication — fundamentally different from consumer Internet.

- Standardization organizations addressing IIoT: IETF, ETSI, W3C, IEEE, IEC.

- Key requirements: reliability, determinism, security, scalability.

## Chapter 3 — Interoperability

### 3.1 Applications in the IoT

- Heterogeneous devices, protocols, and data formats create interoperability challenges.

- Cloud-based vertical solutions (Section 3.2) address integration but risk vendor lock-in.

### 3.2 The Verticals: Cloud-based Solutions

- Vertical silos: Each IoT application domain has proprietary platforms — no horizontal integration.

- Goal: Move toward a horizontal, interoperable IoT architecture with shared semantics and access mechanisms.

### 3.3 REST Architectures: The Web of Things

#### 3.3.1 REST: The Web as a Platform

- Resource-Oriented Architectures (ROA): Web architecture wherein every entity is a named resource addressable by a URI.

- REST (Representational State Transfer): Architectural style for distributed hypermedia systems, defined by Roy Fielding. Core constraints:

  - Client-Server: Separation of user interface from data storage

  - Statelessness: Each request contains all information needed; no session state stored on server

  - Cacheability: Responses marked as cacheable or non-cacheable

  - Uniform Interface: Standardized method of communication (key REST differentiator)

  - Layered System: Client cannot tell whether it connects directly to end server

  - Code on Demand (optional): Server can extend client functionality via executable code

- Resource Identifiers (URIs): Every resource has a unique, dereferenceable URI.

- Statelessness: Session state maintained entirely on client; each request is self-contained. Improves:

  - Visibility (monitoring systems can interpret any request in isolation)

  - Reliability (easier recovery from partial failures)

  - Scalability (no session affinity required on servers)

- Applications as Finite-State Machines: Application behavior modeled as state transitions driven by resource representations.

- HATEOAS (Hypermedia as the Engine of Application State): Clients discover available actions through hypermedia links in responses — eliminates need for out-of-band documentation of API endpoints.

#### 3.3.2 Richardson Maturity Model (RMM)

A model to assess the degree of REST compliance of a web service, with four levels:

- Level 0 — Swamp of POX (Plain Old XML): Single URI, single HTTP method (usually POST); no HTTP semantics used; RPC-style.

- Level 1 — Resources: Multiple URIs (one per resource); still single method.

- Level 2 — HTTP Verbs: Multiple URIs + correct HTTP methods; status codes carry meaning; improves cacheability and proxy compatibility.

- Level 3 — Hypermedia (HATEOAS): Responses include hyperlinks to available state transitions; full REST; self-describing API.

### 3.4 The Web of Things (WoT)

- Definition: An architectural approach that integrates IoT devices into the World Wide Web using standard web protocols and REST principles.

- Goal: Making IoT resources accessible via standard web interfaces (HTTP/CoAP), enabling web-based application development over physical devices.

- Smart objects expose their capabilities as REST resources, described using link formats and media types.

### 3.5 Messaging Queues and Publish/Subscribe Communications

#### 3.5.1 Advantages of Pub/Sub Model

- Decoupling: Publishers and subscribers are independent — neither needs to know the other's identity or address.

- Scalability: Broker handles fan-out; publishers send once.

- Asynchronous operation: Publishers and subscribers operate independently in time.

- Event-driven: Natural fit for sensor-driven IoT applications.

#### 3.5.2 Disadvantages of Pub/Sub Model

- Broker dependency: Central broker is a single point of failure.

- Delivery guarantees: Ensuring exactly-once delivery is complex.

- Latency: Broker intermediation introduces additional hop.

#### 3.5.3 MQTT (Message Queue Telemetry Transport)

- Definition: Lightweight publish/subscribe messaging protocol designed for constrained devices and low-bandwidth, high-latency networks.

- Transport: Runs over TCP (not UDP), ensuring reliable delivery.

- Quality of Service (QoS) levels:

  - QoS 0 (At most once): Fire-and-forget; no acknowledgment

  - QoS 1 (At least once): Message delivered at least once; ACK required; duplicates possible

  - QoS 2 (Exactly once): Four-way handshake ensures exactly-once delivery; highest overhead

- Topics: Hierarchical slash-separated strings (e.g., home/livingroom/temperature); wildcards: + (single level), # (multi-level).

- Broker: Central component managing subscriptions and routing messages.

- Retained messages: Broker stores last message per topic; new subscribers receive it immediately.

- Will messages: Pre-configured message sent by broker on behalf of a client upon unexpected disconnection.

MQTT vs. AMQP:

- MQTT: simpler, lighter, better for highly constrained clients; less interoperability at protocol level.

- AMQP: richer semantics, more complex, better for enterprise integration.

### 3.6 Session Initiation for the IoT

#### 3.6.1 Motivations

- IoT devices require establishing communication sessions for data exchange — analogous to SIP in VoIP but adapted for constrained environments.

- Classical SIP is too heavyweight for constrained devices (XML-based, text messages, requires TCP/TLS).

#### 3.6.2 Lightweight Sessions in the IoT — CoAP-based Session Protocol

Architecture Entities:

- UAC (User Agent Client): Initiates session requests

- UAS (User Agent Server): Responds to session requests

Session Description: Uses Content-Format CoAP option to specify format of offer/answer.

Session Initiation Procedure (4-message exchange):

- UAC → UAS: CON POST with Observe = 0 (session offer in payload)

- UAS → UAC: ACK (confirms receipt)

- UAS → UAC: CON 2.01 Created (Location = session_id; session answer in payload)

- UAC → UAS: ACK (confirms receipt) → Session established; data exchange begins.

Session Tear-down: Either party sends a request terminating the session; partner frees processing/memory resources. Graceful termination preferred over implicit timeout.

Session Modification: Renegotiation without full re-establishment.

### 3.7 Performance Evaluation of Session Initiation

- Implementation: Prototype on constrained hardware (TelosB/Zolertia motes) running ContikiOS with CoAP stack.

- Experimental results: Session setup overhead is minimal; CoSIP outperforms classical SIP for constrained devices in terms of message size and processing overhead.

### 3.8 Optimized Communications: Dual-Network Management Protocol (DNMP)

#### 3.8.1 Motivations

- IoT networks often combine heterogeneous radio technologies — low-power/low-throughput (e.g., IEEE 802.15.4) and high-power/high-throughput (e.g., IEEE 802.11s mesh).

- Challenge: Use low-power network for control and selectively activate the high-power network for data-intensive transfers.

#### 3.8.2 The DNMP Protocol

- Dual-network architecture:

  - LPLT (Low-Power Low-Throughput) network: IEEE 802.15.4 — always-on control channel

  - HPHT (High-Power High-Throughput) network: IEEE 802.11s mesh — selectively activated

- Node integration: Nodes equipped with both radio interfaces; LPLT is used for management; HPHT is activated on demand.

- IEEE 802.15.4-controlled selective activation: LPLT network carries control messages triggering the activation/deactivation of the IEEE 802.11s network.

#### 3.8.3 Performance Evaluation

- Experimental setup: Real testbed with IEEE 802.15.4 + IEEE 802.11s nodes.

- Results: DNMP achieves significant energy savings by keeping HPHT radio dormant until needed; LPLT control overhead is minimal.

- Operational limitations of IEEE 802.15.4: Low duty-cycle operation introduces latency in control signaling.

### 3.9 Discoverability in Constrained Environments

#### 3.9.1 CoRE Link Format (RFC 6690)

- Purpose: Provides a standard format to describe IoT resources hosted on constrained servers, enabling clients to discover available resources.

- Discovery mechanism: Client sends GET to /.well-known/core URI; server returns CoRE Link Format document.

- Link Format: Comma-separated list of links; each link enclosed in angle brackets (<>); attributes follow separated by semicolons.

  - Example: </sensors/temp>;rt="TemperatureC";if="sensor";obs

- Interface Description (if) attribute: Describes the functional interface type of a resource.

#### 3.9.2 CoRE Interfaces

Standard interface types for IoT resources:

| Interface | Description |
| --- | --- |
| Sensor | Read-only resource representing a sensor measurement |
| Parameter | Read/write resource representing a configurable parameter |
| Read-only Parameter | Parameter that can only be read |
| Actuator | Write resource that triggers physical actions |

### 3.10 Data Formats: SenML (Sensor Markup Language)

- Purpose: Standard JSON/XML format for representing sensor measurements and device attributes.

- Structure: Uses measurement records with fields:

  - bn (Base Name), bt (Base Time), bu (Base Unit), bv (Base Value)

  - n (Name), u (Unit), v (Value), vs (String Value), vb (Boolean Value), t (Time), ut (Update Time)

JSON Representations:

- Single Datapoint: Single measurement object {"n":"temperature","u":"Cel","v":25.3}

- Multiple Datapoints: Array of measurement objects for same sensor at different times

- Multiple Measurements: Array spanning multiple sensors/resources

## Chapter 4 — Discoverability

### 4.1 Service and Resource Discovery

- Definition: The mechanism by which IoT entities locate services, resources, and other devices without prior knowledge of their addresses.

- Two scales:

  - Local (small-scale): Within a subnet or building

  - Large-scale: Across geographically distributed networks

### 4.2 Local and Large-Scale Service Discovery

#### 4.2.1 ZeroConf (Zero Configuration Networking)

- Purpose: Enables automatic network configuration without manual setup or DHCP/DNS servers.

- Components:

  - Link-local addressing: Automatic IP address assignment (169.254.0.0/16 for IPv4; fe80::/10 for IPv6)

  - Multicast DNS (mDNS): DNS resolution via multicast on local link (port 5353)

  - DNS Service Discovery (DNS-SD): Service registration and discovery using standard DNS record types (PTR, SRV, TXT)

#### 4.2.2 UPnP (Universal Plug and Play)

- Purpose: Automatic device discovery and service description in local networks.

- Phases: Discovery (SSDP multicast), Description (XML via HTTP), Control (SOAP), Eventing (subscriptions).

- Limitation: Heavyweight for constrained IoT devices.

#### 4.2.3 URI Beacons and the Physical Web

- URI Beacons: BLE advertising packets that broadcast a URL; nearby devices receive and can interact with the advertised resource.

- Physical Web: Google initiative enabling URLs to be broadcast from physical objects via BLE beacons; users can discover nearby objects through their smartphones.

### 4.3 Scalable and Self-Configuring Architecture for Service Discovery

#### 4.3.1 IoT Gateway

- Proxy Functionality: Translates between protocols (e.g., CoAP ↔ HTTP); manages constrained devices from the Internet.

- Service and Resource Discovery: Gateway maintains a resource directory — a registry of resources from attached constrained devices; clients query the gateway rather than individual devices.

#### 4.3.2 P2P-based Large-Scale Service Discovery Architecture

Distributed Location Service:

- Uses Distributed Hash Tables (DHTs) to store and retrieve resource/service records.

- Scales to millions of devices without centralized directory.

Distributed Geographic Table (DGT):

- Maps geographic coordinates to node identifiers in the P2P overlay.

- Enables location-aware service discovery.

Large-Scale P2P Architecture:

- IoT devices register resources with local gateway.

- Gateway announces resources to P2P overlay.

- Remote clients query P2P overlay and are directed to the appropriate gateway.

- Architecture supports autonomic resource registration and deregistration.

#### 4.3.3 Zeroconf-based Local Service Discovery for Constrained Environments

- Architecture: Adapted mDNS/DNS-SD for constrained IEEE 802.15.4 networks.

- Service Discovery Protocol: Multicast-based, but with adaptations for low-power duty-cycling.

- Key challenge: Standard multicast not efficiently supported in duty-cycled networks; solutions introduced below.

#### 4.3.4 Implementation Results

- Local Service Discovery: Validated on real ContikiOS/IEEE 802.15.4 testbed.

- Large-Scale Discovery: Validated using P2P overlay simulation with thousands of nodes.

### 4.4 Lightweight Service Discovery in Low-Power IoT Networks

#### 4.4.1 Efficient Forwarding Protocol for Service Discovery

Multicast through Local Filtered Flooding:

- Constrained multicast: sender broadcasts DNS-SD query; intermediate nodes selectively forward based on filtering criteria.

- Local Filtered Flooding: Nodes forward messages only if relevant to their own service registry; reduces unnecessary transmissions.

Efficient Multiple Unicast Forwarding:

- DNS-SD query sent as individual unicast messages to known neighbor nodes.

- Nodes aggregate and forward responses.

- Comparison vs. broadcast:

  - At low Channel Check Rate (CCR): multiple unicast outperforms broadcast (ContikiMAC broadcast occupies channel for entire wake-up interval).

  - At high CCR (≈128 Hz, near null duty-cycling): broadcast becomes competitive.

  - Energy consumption: broadcast far less efficient than multiple unicast in all typical scenarios; border router (no duty-cycling) contributes >60% of energy with multiple unicast, >30% with broadcast.

### 4.5 Implementation Results

- Extensive experimentation in linear and grid topologies.

- Metrics: Query Completion Rate (QCR), energy consumption per query, latency.

- Key finding: Multiple unicast forwarding is energetically superior to broadcast for typical IoT CCR values; both approaches converge at very high CCR.

## Chapter 5 — Security and Privacy in the IoT

### 5.1 Security Issues in the IoT

- IoT security threats span all layers of the protocol stack.

- Primary threat categories:

  - Physical attacks: Node capture, hardware tampering

  - Link-layer attacks: Jamming, eavesdropping, replay attacks

  - Network-layer attacks: Routing attacks (blackhole, wormhole, sinkhole), Sybil attacks

  - Transport-layer attacks: Session hijacking

  - Application-layer attacks: Data injection, firmware modification

  - Privacy attacks: Traffic analysis, location tracking

- Challenge: Classical security mechanisms (TLS, AES-256, RSA-2048) are too computationally and memory-intensive for severely constrained devices.

### 5.2 Security Mechanisms Overview

#### 5.2.1 Traditional vs. Lightweight Security

Network Layer:

- IPSec (traditional): Provides network-layer security via Authentication Header (AH) and Encapsulating Security Payload (ESP). Mandatory in IPv6.

  - AH: Provides integrity and authentication; no encryption

  - ESP: Provides confidentiality, integrity, and authentication

  - Key exchange via IKE (Internet Key Exchange)

- Lightweight adaptation: Constrained devices may implement only essential IPSec components with reduced cryptographic overhead.

Transport Layer:

- TLS (Transport Layer Security): Provides confidentiality, integrity, and authentication over TCP.

- DTLS (Datagram TLS): TLS adapted for UDP (retransmission, reordering, no connection state). Mandatory security mechanism for CoAP.

  - Adds handshake overhead but much lighter than TLS due to stateless UDP nature.

Application Layer:

- S/MIME: Secure MIME for email/messaging encryption.

- OAuth: Authorization framework enabling third-party access without credentials sharing; adapted for IoT (IoT-OAS).

#### 5.2.2 Lightweight Cryptography (LWC)

Symmetric-Key LWC Algorithms: Designed to operate on microcontrollers with very limited resources (8/16/32-bit MCUs, < 2 KB RAM):

| Algorithm | Block Size (bits) | Key Size (bits) | Rounds | Target |
| --- | --- | --- | --- | --- |
| PRESENT | 64 | 80 or 128 | 31 | Ultra-constrained hardware |
| SIMON | 32–128 | 64–256 | Varies | Flexible, hardware-optimized |
| SPECK | 32–128 | 64–256 | Varies | Software-optimized |
| CLEFIA | 128 | 128/192/256 | 18/22/26 | Smart cards, IoT |
| KATAN/KTANTAN | 32/48/64 | 80 | 254 | Hardware only |
| HIGHT | 64 | 128 | 32 | Low-resource hardware |

AES (standard): 128-bit block, 128/192/256-bit keys — computationally feasible on modern constrained devices via hardware acceleration; serves as reference for LWC comparison.

Public-Key (Asymmetric) LWC Algorithms: Classical RSA is too computationally expensive for constrained devices. Alternatives:

- Elliptic Curve Cryptography (ECC):

  - Based on the algebraic structure of elliptic curves over finite fields.

  - Security equivalent: ECC-160 bit ≈ RSA-1024 bit security, with far smaller key sizes.

  - ECC equation: Points (x, y) satisfying y² = x³ + ax + b (mod p) for prime p, with 4a³ + 27b² ≠ 0.

  - Operations: Point addition and scalar multiplication; private key = integer k; public key = kP (P is a base point).

  - ECDH (Elliptic Curve Diffie-Hellman): Key agreement protocol; efficient for constrained devices.

  - ECDSA (Elliptic Curve Digital Signature Algorithm): Digital signatures with smaller signatures than RSA.

  - Standardized curves: NIST P-256, Curve25519, Brainpool curves.

- NTRU: Lattice-based public-key cryptosystem; post-quantum candidate; faster than RSA for constrained devices.

Lightweight Cryptographic Hash Functions:

| Algorithm | Output (bits) | Block (bits) | Rounds | Notes |
| --- | --- | --- | --- | --- |
| SHA-1 | 160 | 512 | 80 | Legacy; vulnerable to collision attacks |
| SHA-256 | 256 | 512 | 64 | Standard; feasible on constrained devices |
| PHOTON | 80–288 | — | 12 | Ultra-lightweight; hardware-oriented |
| SPONGENT | 88–256 | — | Varies | Sponge-based; hardware-optimized |
| QUARK | 136/176/256 | — | Varies | Very low area |

Homomorphic Encryption Schemes:

- Definition: Encryption schemes allowing computations on ciphertext that, when decrypted, yield the same result as if operations were performed on plaintext.

- Fully Homomorphic Encryption (FHE): Supports arbitrary computations; currently too computationally expensive for IoT.

- Partially Homomorphic Encryption (PHE): Supports limited operations:

  - Additive homomorphic: E(a) ⊕ E(b) = E(a + b) — e.g., Paillier cryptosystem

  - Multiplicative homomorphic: E(a) ⊗ E(b) = E(a × b) — e.g., RSA, ElGamal

- IoT application: Cloud can aggregate encrypted sensor data without decryption — preserves privacy while enabling analytics.

#### 5.2.3 Key Agreement, Distribution, and Security Bootstrapping

Key Agreement Protocols:

- Diffie-Hellman (DH): Enables two parties to establish a shared secret over an insecure channel without prior shared secrets.

  - Classical DH based on modular exponentiation; ECDH is the constrained-friendly variant.

- Pre-shared Keys (PSK): Both parties pre-configured with a shared key; simpler but less scalable.

Shared Group-Key Distribution:

- Problem: Distributing a group key to multiple constrained nodes efficiently.

- Solutions: Key distribution center (KDC), pairwise key establishment followed by rekeying, broadcast encryption.

Security Bootstrapping:

- Challenge: New devices joining an IoT network must securely establish initial credentials.

- Mechanisms: Factory-provisioned certificates, zero-touch provisioning protocols, trust anchor distribution.

- DTLS-based bootstrapping: Uses pre-shared keys or certificates to establish DTLS session; credentials derived or updated during session.

#### 5.2.4 Secure Data Aggregation

- Purpose: In-network aggregation reduces communication overhead, but if aggregator is compromised, data integrity is lost.

- Approach: Combine data aggregation with cryptographic integrity: nodes encrypt individual readings; aggregator combines without decrypting (using additive homomorphic encryption).

- Result: Base station decrypts and obtains the aggregated result without intermediate nodes seeing plaintext.

#### 5.2.5 Authorization Mechanisms for Secure IoT Services

- Access control: Determining which entities can access which resources and under what conditions.

- Models:

  - ACL (Access Control Lists): Per-resource list of permitted entities

  - RBAC (Role-Based Access Control): Access granted based on roles

  - ABAC (Attribute-Based Access Control): Fine-grained policies based on attributes

- OAuth 2.0: Token-based authorization framework; access tokens grant specific permissions.

### 5.3 Privacy Issues in the IoT

#### 5.3.1 The Role of Authorization

- Privacy threat: IoT devices continuously collect personal data; unauthorized access violates privacy.

- Authorization as privacy protection: Proper authorization ensures only entitled entities access personal data.

- Delegation: Authorization can be delegated — resource owner delegates access rights to a third party without sharing credentials.

#### 5.3.2 IoT-OAS: Delegation-based Authorization for the Internet of Things

Architecture:

- Extends OAuth 2.0 for constrained IoT environments.

- Components:

  - Resource Owner: Controls access to IoT resources

  - Client (Requesting Party): Requests access to protected resources

  - IoT-OAS Server: OAuth Authorization Server adapted for IoT

  - Service Provider (SP): Hosts protected IoT resources

Granting Access Tokens:

- Client requests access token from IoT-OAS with resource description.

- IoT-OAS verifies Client's identity and Resource Owner's policy.

- IoT-OAS issues a token encoding permitted operations and target resources.

Authorizing Requests:

- Client presents token to SP when accessing resource.

- SP validates token with IoT-OAS (or locally if token is self-contained).

- Access granted or denied based on token validity and permissions.

SP-to-IoT-OAS Communication: Protocol details for token introspection/validation via CoAP or HTTP.

#### 5.3.3 IoT-OAS Application Scenarios

| Scenario | Description |
| --- | --- |
| Network Broker Communication | Authorization via intermediate broker |
| Gateway-based Communication | Gateway enforces access control on behalf of constrained devices |
| End-to-End CoAP Communication | Direct client-to-device authorization with tokens |
| Hybrid Gateway-based | Combines gateway and direct authorization |

## Chapter 6 — Cloud and Fog Computing for the IoT

### 6.1 Cloud Computing

- Definition: On-demand availability of computing resources (servers, storage, databases, networking, software) over the Internet.

- Service models: IaaS (Infrastructure as a Service), PaaS (Platform as a Service), SaaS (Software as a Service).

- Role in IoT: Provides scalable storage, processing, and analytics for IoT-generated data.

### 6.2 Big Data Processing Pattern

- Big Data characteristics (5 Vs): Volume, Velocity, Variety, Veracity, Value.

- Processing paradigms:

  - Batch processing: Large data volumes processed at intervals (Hadoop MapReduce)

  - Stream processing: Continuous real-time processing of data in motion

- IoT data is predominantly stream-oriented (continuous sensor readings) → Big Stream processing required.

### 6.3 Big Stream

#### 6.3.1 Big-Stream-Oriented Architecture

- Big Stream: Refers to the processing of high-velocity, continuous IoT data streams from large numbers of sensors.

- Architecture components:

  - Acquisition Module: Receives data from heterogeneous IoT sources (supporting MQTT, HTTP, CoAP); normalizes data format.

  - Normalization Module: Converts heterogeneous data into a common representation (SenML-based).

  - Graph Framework: Processes normalized data streams using directed acyclic graphs (DAGs) where nodes represent processing operations.

  - Application Register Module: Manages registration of stream processing applications; routes data to appropriate applications.

#### 6.3.2 Graph-Based Processing

- Directed Acyclic Graph (DAG) model: Each processing step is a graph node; data flows along directed edges.

- Node types: Source (data ingestion), Processing (transformation, filtering, aggregation), Sink (output/storage).

- Benefits: Parallel processing of multiple streams; modular composition of processing pipelines; easy visualization of data flow.

#### 6.3.3 Implementation

- Acquisition Module: Implements protocol-specific adapters for MQTT (TCP-based), HTTP, and CoAP.

- Normalization Module: Translates proprietary formats to SenML JSON; resolves unit inconsistencies.

- Graph Framework: Built on top of a distributed stream processing engine; distributes graph nodes across computing resources.

- Application Register Module: Allows dynamic registration/deregistration of applications; persists registration metadata.

#### 6.3.4 Performance Evaluation

- Metrics: Throughput (messages/second), latency (end-to-end delay), scalability (performance vs. number of devices/streams).

- Results: The architecture sustains high-throughput processing with manageable latency; graph-based processing allows horizontal scaling.

### 6.4 Big Stream and Security

#### 6.4.1 Graph-Based Cloud System Security

- Threat model: Untrusted data sources, compromised graph nodes, unauthorized application access.

- Security requirements: Data authenticity, confidentiality, access control at the stream level.

#### 6.4.2 Normalization after Secure Stream Acquisition (OFS Module)

- OFS (OAuth-secured Frontend for Streams): Authenticates data providers via OAuth before accepting data streams.

- Protocol-specific: OAuth integrated with MQTT, HTTP, and CoAP adapters.

#### 6.4.3 Enhancing the Application Register with the IGS Module

- IGS (IoT Graph Security) Module: Enforces security policies at the graph level.

- Functions: Authenticate applications registering for stream data; authorize data access based on security rules; filter unauthorized stream subscriptions.

- Security rules engine (Ψ): Evaluates rules against graph node attributes and application credentials; grants (GRAN) or denies processing.

- PMSV (Policy Management and Security Verification): Manages and verifies security policies.

#### 6.4.4 Securing Streams Inside Graph Nodes

- Inter-node encryption: Data encrypted between graph nodes using session keys derived during registration.

- Integrity verification: Cryptographic MACs appended to stream data within the graph.

#### 6.4.5 Evaluation of a Secure Big Stream Architecture

- Overhead analysis: Security mechanisms add latency; DTLS for CoAP adds ~20–40 ms handshake.

- Throughput impact: Encryption/decryption within graph nodes reduces throughput by ~15–25% depending on cipher; AES-GCM offers best throughput/security tradeoff.

### 6.5 Fog Computing and the IoT

- Definition (NIST): Computing infrastructure located at the network edge, between end devices and the cloud.

- Difference from Cloud: Fog nodes are geographically distributed, closer to devices; lower latency; local data processing.

- IoT relevance:

  - Reduces cloud bandwidth requirements by pre-processing data locally

  - Enables real-time local actuation decisions

  - Supports offline operation when cloud is unavailable

  - Reduces privacy exposure by keeping sensitive data local

### 6.6 The Role of the IoT Hub

#### 6.6.1 Virtualization and Replication

The IoT Hub:

- Definition: A software entity that acts as a virtual representation (digital twin) of one or more IoT devices in the cloud or fog layer.

- Functions:

  - Maintains the current state of associated devices (virtualization)

  - Serves requests for device state without requiring direct device communication (reduces device wake-up frequency → energy saving)

  - Replicates state across multiple fog nodes for availability

Operational Scenarios:

- Local read: Client queries IoT Hub; Hub serves cached device state; device not contacted.

- State update: Device reports new state to Hub; Hub propagates update to interested clients.

- Actuation: Client sends command to Hub; Hub queues command and delivers to device when online.

- Federation: Multiple IoT Hubs synchronize state across geographic locations.

Synchronization Protocol:

- Challenge: Maintaining consistency of device state across replicated Hub instances.

- Protocol design:

  - Primary Hub: Authoritative state; receives direct device updates.

  - Secondary Hubs (replicas): Subscribe to Primary for state changes.

  - Consistency model: Eventual consistency — replicas converge to Primary state after propagation delay.

  - Conflict resolution: Timestamp-based; most recent update wins.

  - Message format: Uses CoAP or MQTT for inter-Hub synchronization.

- Synchronization messages include: Device ID, resource URI, current value, timestamp, version counter.

## Chapter 7 — The IoT in Practice

### 7.1 Hardware for the IoT

#### 7.1.1 Classes of Constrained Devices (RFC 7228)

| Class | Code Size | RAM | Examples |
| --- | --- | --- | --- |
| Class 0 | < 10 KB | < 1 KB | Simple sensors, RFID tags |
| Class 1 | ~10 KB | ~1 KB | TelosB, Zolertia Z1 |
| Class 2 | ~50 KB | ~10 KB | Arduino, more capable motes |

- Classes 0 and 1 are the primary targets of CoAP, 6LoWPAN, and LWC algorithms.

- Class 2 devices can support more conventional protocols with optimization.

#### 7.1.2 Hardware Platforms

##### TelosB

- MCU: TI MSP430, 16-bit RISC, 8 MHz, 10 KB RAM, 48 KB flash

- Radio: IEEE 802.15.4 (2.4 GHz), 250 kbps

- Sensors: Temperature, humidity, light (photosynthetically active radiation, total solar radiation)

- OS support: TinyOS, ContikiOS

- Use case: Reference platform for WSN/IoT research; widely validated.

##### Zolertia Z1

- MCU: TI MSP430F2617, 16 MHz, 8 KB RAM, 92 KB flash

- Radio: IEEE 802.15.4 (2.4 GHz), CC2420 transceiver, 250 kbps

- Sensors: 3-axis accelerometer (ADXL345), temperature/humidity (TMP102)

- Interfaces: UART, I²C, SPI

- OS support: ContikiOS, TinyOS, RIOT

- Use case: Advanced constrained IoT node; suitable for 6LoWPAN/CoAP experiments.

##### OpenMote

- MCU: TI CC2538 (ARM Cortex-M3 at 32 MHz), 32 KB RAM, 512 KB flash

- Radio: IEEE 802.15.4 (2.4 GHz), CC2538 integrated transceiver, 250 kbps

- Additional radio: Optional CC1200 sub-GHz radio for long-range applications

- OS support: OpenWSN, ContikiOS, RIOT

- Use case: High-end constrained node supporting 6TiSCH (IEEE 802.15.4e TSCH scheduling).

##### Arduino

- MCU: ATmega328P (Arduino Uno), 8-bit, 16 MHz, 2 KB SRAM, 32 KB flash

- Ecosystem: Extensive shield ecosystem for sensors, actuators, connectivity modules

- Connectivity: Via shields (Ethernet, Wi-Fi, ZigBee, BLE)

- OS: Bare metal (Arduino IDE); RTOS integrations possible

- Use case: Rapid prototyping; not suitable for full IP stack without co-processor.

##### Intel Galileo

- MCU: Intel Quark SoC X1000, 32-bit, 400 MHz, 256 MB RAM, 8 MB flash

- Interfaces: Full PCIe, USB, Ethernet, mini-PCIe, GPIO

- OS: Linux (Yocto Project)

- Arduino compatibility: Arduino shield-compatible form factor

- Use case: Gateway-class device; capable of running full IP stack, cloud connectivity.

##### Raspberry Pi

- MCU: ARM Cortex-A (varies by model); Raspberry Pi 3: 4× ARM Cortex-A53 at 1.2 GHz, 1 GB RAM

- Connectivity: Gigabit Ethernet, Wi-Fi (802.11 b/g/n/ac), Bluetooth 4.2/BLE (Model 3B+)

- OS: Raspbian (Debian-based Linux), Ubuntu, others

- GPIO: 40-pin GPIO for sensor/actuator interfacing

- Use case: IoT hub/gateway; edge computing node; Web of Things platform host.

### 7.2 Software for the IoT

#### 7.2.1 OpenWSN

- Type: Open-source implementation of the full IEEE 802.15.4e/6TiSCH/CoAP/6LoWPAN stack.

- Target: Platforms supporting Time-Slotted Channel Hopping (TSCH).

- Features: Deterministic scheduling, low power, high reliability.

#### 7.2.2 TinyOS

- Type: Event-driven operating system for constrained sensor nodes.

- Language: nesC (component-based C extension).

- Concurrency model: Tasks (deferred execution) + interrupt handlers (split-phase I/O).

- Memory model: Static allocation; no dynamic memory allocation.

- IoT support: 6LoWPAN, CoAP via Blip networking stack.

#### 7.2.3 FreeRTOS

- Type: Real-time operating system (RTOS) for embedded systems.

- Features: Preemptive scheduling, message queues, semaphores, software timers.

- Footprint: Kernel can fit in < 6 KB of RAM.

- Connectivity: FreeRTOS+TCP provides TCP/IP stack; MQTT via FreeRTOS+MQTT.

- Target: Class 1–2 devices; wide MCU support (ARM Cortex-M, AVR, PIC, etc.)

#### 7.2.4 TI-RTOS

- Type: TI's proprietary RTOS for TI MCUs and DSPs.

- Features: Power management, peripheral drivers, DMA support.

- Connectivity: SimpleLink Wi-Fi, BLE, ZigBee SDKs.

- Target: TI MSP430, CC2xxx, Tiva MCU families.

#### 7.2.5 RIOT

- Type: Micro-kernel OS for IoT; designed for Class 1 and 2 devices.

- Architecture: Micro-kernel with drivers, network stack, and application layer.

- Networking: Full 6LoWPAN, CoAP (nanocoap/gcoap), DTLS, MQTT.

- Concurrency model: Multi-threading with preemptive scheduling.

- Key feature: POSIX-compatible API; C/C++ support.

#### 7.2.6 ContikiOS

- Type: Open-source, portable, event-driven OS for constrained IoT nodes.

- Creator: Adam Dunkels (SICS); widely used in academic and industrial IoT.

Networking:

- Full uIPv6 stack with 6LoWPAN, RPL (IPv6 Routing Protocol for Low-power and Lossy Networks), CoAP, DTLS, MQTT support.

- RPL: Distance-vector routing protocol for LLNs; builds a Destination-Oriented DAG (DODAG) rooted at a border router.

Low-Power Operation:

- ContikiMAC: Radio duty cycling protocol; nodes periodically wake up to check for transmissions; sender transmits repeatedly until receiver wakes and acknowledges.

- X-MAC, TDMA modes also supported.

- Typical duty cycle: < 1%.

Simulation:

- Cooja simulator: Network simulator integrated with ContikiOS; runs actual node firmware in simulation; supports IEEE 802.15.4 radio medium emulation.

- Enables testing of protocols on virtual networks of hundreds of nodes before physical deployment.

Programming Model:

- Protothreads: Lightweight stackless threads implemented using C macros; enable sequential programming style within event-driven model.

- Process abstraction: Each ContikiOS process is a protothread; scheduled cooperatively.

- Events: Inter-process communication via posted events.

Features:

- Coffee File System (CFS): lightweight flash-based file system.

- ELF loader: enables dynamic code loading.

- Power profiling tools.

- Widely ported: TelosB, Zolertia Z1, OpenMote, sky, native (Linux/Windows simulation).

### 7.3 Vision and Architecture of a Testbed for the Web of Things

#### 7.3.1 All-IP-based Infrastructure for Smart Objects

- Architecture: Full IP connectivity from edge sensor nodes to Internet, via 6LoWPAN border routers and standard IP routing.

- Access model: Each smart object accessible via a URI; no protocol translation needed beyond 6LoWPAN/IPv6 boundary.

#### 7.3.2 Enabling Interactions with Smart Objects through the IoT Hub

- IoT Hub role: Acts as a proxy/gateway between Internet clients and constrained devices.

- Integration challenges:

  - Protocol translation (HTTP ↔ CoAP)

  - Addressing (IPv6 ↔ 6LoWPAN shortened addresses)

  - Security policy enforcement

  - Device state virtualization (clients query Hub instead of device)

#### 7.3.3 Testbed Access and Security

- Authorization: IoT-OAS framework enforces access control at the testbed level.

- Role of authorization: Prevents unauthorized access to testbed resources; enables multi-tenant operation.

#### 7.3.4 Exploiting the Testbed: WoT Applications for Mobile and Wearable Devices

- Demonstration applications: Mobile apps interacting with physical sensors via REST APIs.

- Wearable integration: Smartwatches and fitness trackers as IoT clients; interact with smart objects in the testbed environment.

#### 7.3.5 Open Challenges and Future Vision

- Scalability: Supporting millions of devices per deployment.

- Heterogeneity: Seamless integration of diverse radio technologies.

- Autonomicity: Self-configuring, self-healing deployments.

- Standards evolution: Tracking evolving IETF and IEEE standards.

### 7.4 Wearable Computing for the IoT

#### 7.4.1 Shaping the IoT in a Mobile-Centric World

- Mobile devices (smartphones, wearables) as primary user interfaces to IoT.

- Mobile-centric paradigm: Smart objects exposed as REST resources accessible from mobile apps.

#### 7.4.2 Interaction Patterns with Smart Objects through Wearable Devices

Smart Object Communication Principles:

- Stateless REST interactions; resources identified by URIs.

- Content negotiation via Content-Format option.

- Observe pattern for real-time updates from sensors.

Interaction Patterns:

| Pattern | Description | CoAP Mechanism |
| --- | --- | --- |
| Read | Retrieve current state of resource | GET |
| Write | Update resource state (configuration/actuator) | PUT/POST |
| Observe | Continuously receive resource state updates | GET + Observe option |
| Create | Create a new resource | POST |
| Delete | Remove a resource | DELETE |

#### 7.4.3 Implementation in a Real-World IoT Testbed

- Prototype implemented with Android wearable (smartwatch) + ContikiOS sensor nodes.

- Communication: Android ↔ IoT Hub (HTTP/REST) ↔ CoAP proxy ↔ constrained sensor nodes.

- Future vision toward Tactile Internet: Ultra-low latency IoT enabling haptic feedback and remote physical interaction.

### 7.5 Effective Authorization for the Web of Things

#### 7.5.1 Authorization Framework Architecture

System Components:

- Resource Server (RS): Hosts IoT resources; enforces access control.

- Authorization Server (AS): Issues access tokens (IoT-OAS-based).

- Client: Mobile/web application requesting access to IoT resources.

- Resource Owner: Human or system controlling access policy.

System Operations:

- Client authenticates with AS and requests access token for target resource(s).

- AS verifies client identity and checks access policy defined by Resource Owner.

- AS issues an access token (JWT or CoAP-specific token format) encoding:

  - Client identity

  - Target resource(s) URI

  - Permitted operations (GET/PUT/POST/DELETE)

  - Token lifetime

- Client presents token to RS with each request.

- RS validates token (locally or via AS introspection) and grants/denies access.

#### 7.5.2 Implementation and Validation

- Implemented on real testbed using Android clients, Raspberry Pi IoT Hub, and ContikiOS sensor nodes.

- Validation metrics: Token issuance latency, authorization decision latency, overhead on constrained devices.

- Results: IoT-OAS adds < 50 ms overhead for gateway-enforced authorization; direct device authorization requires DTLS session (adds ~200–400 ms for initial handshake but amortizes over session duration).

## Summary of Key Standards and Protocols Referenced

| Standard/Protocol | Layer | Purpose | Specification |
| --- | --- | --- | --- |
| IEEE 802.3 | PHY/MAC | Wired Ethernet LAN | IEEE |
| IEEE 802.11 | PHY/MAC | Wi-Fi WLAN | IEEE |
| IEEE 802.15.4 | PHY/MAC | LR-WPAN (ZigBee/6LoWPAN base) | IEEE |
| IPv4 | Network | 32-bit Internet addressing | RFC 791 |
| IPv6 | Network | 128-bit Internet addressing | RFC 2460 |
| 6LoWPAN | Adaptation | IPv6 over IEEE 802.15.4 | RFC 4944, 6282 |
| RPL | Network | Routing in Low-power Lossy Networks | RFC 6550 |
| TCP | Transport | Reliable, ordered, connection-based | RFC 793 |
| UDP | Transport | Unreliable, connectionless, lightweight | RFC 768 |
| DTLS | Transport | Datagram TLS (UDP security) | RFC 6347 |
| HTTP | Application | Web request/response | RFC 7230–7235 |
| CoAP | Application | REST for constrained IoT nodes | RFC 7252 |
| CoAP Observe | Application | Resource state subscription | RFC 7641 |
| CoAP Block | Application | Blockwise payload transfer | RFC 7959 |
| MQTT | Application | Lightweight pub/sub messaging | OASIS |
| AMQP | Application | Message-oriented middleware | OASIS |
| SIP | Application | Session initiation (VoIP/multimedia) | RFC 3261 |
| CoRE Link Format | Application | Resource description/discovery | RFC 6690 |
| SenML | Application | Sensor data markup language | IETF Draft |
| mDNS/DNS-SD | Discovery | Local service discovery | RFC 6762/6763 |
| OAuth 2.0 | Security | Authorization framework | RFC 6749 |
| IPSec (AH/ESP) | Security | Network-layer security | RFC 4301 |
| TLS | Security | Transport-layer security | RFC 5246/8446 |
| ECC/ECDH/ECDSA | Security | Lightweight asymmetric cryptography | NIST |

## Architectural Design Principles (Recurring Themes)

- IP-everywhere: All IoT devices should be addressable via IPv6; 6LoWPAN bridges constrained link layers to the IP world.

- REST as universal interface: CoAP and HTTP with REST principles provide a uniform interaction model across all IoT tiers.

- Constrained-first design: All protocols (CoAP, DTLS, LWC, 6LoWPAN) are designed with severely resource-limited devices (Class 0–1) as the primary target.

- Layered security: Security applied at network (IPSec), transport (DTLS), and application (OAuth/IoT-OAS) layers; no single layer is sufficient.

- Fog/cloud continuum: Processing distributed across device edge (ContikiOS), fog (IoT Hub), and cloud (Big Stream) according to latency, bandwidth, and privacy requirements.

- Discoverability as a first-class concern: CoRE Link Format, DNS-SD, and P2P overlay architectures enable autonomous device discovery at all scales.

- Interoperability over proprietary solutions: Preference for open IETF/IEEE/OASIS standards enables cross-vendor, cross-domain integration.

This summary covers all 7 chapters and their subsections, all key protocols with their precise specifications, all defined message formats, all security algorithms with comparative tables, all hardware platforms with their exact specifications, and all software platforms with their architectural properties — maintaining full technical accuracy as presented in the source text.