### Internet of Things

### Chapter 1: Preliminaries, Motivation, and Related Work

This chapter introduces the fundamental concepts, history, and applications of the Internet of Things (IoT).

#### 1.1 What is the Internet of Things?

- Concept & Purpose: The IoT is a paradigm where billions of everyday objects (smart objects) are augmented with microcontrollers, transceivers, sensors, and actuators to connect over IP (Internet Protocol) networks. Its purpose is to bridge the physical and cyber worlds, moving from a hardware-driven Internet to an open, integrated architecture of interoperable platforms.

- Edge Computing Integration: Introduces processing at the edge of the access network to provide low latency, real-time capabilities, and context-awareness, transforming raw data into Big Data analytics.

#### 1.2 Wireless Ad-hoc and Sensor Networks (WSNs): The Ancestors without IP

- Concept: WSNs were early networks of constrained devices designed without IP integration, relying strictly on ad-hoc, infrastructure-less, cooperative, and multi-hop routing.

- Limitations: Lacked horizontal interoperability; built as closed "vertical" solutions customized only for specific application scenarios.

#### 1.3 IoT-enabled Applications

The adoption of a common IP-oriented stack enables horizontal applications across various domains:

- Home/Building Automation: Security, energy saving, and healthcare monitoring (e.g., for the elderly).

- Smart Cities & Smart Grids: Utilizes Power Line Communications (PLC) on existing electrical cables (3–148 kHz band) to manage energy consumption and automated metering.

- Industrial IoT (IIoT): Demands ultra-reliable, sub-millisecond real-time responses for manufacturing, distinguishing it from the "best effort" consumer Internet.

- Smart Farming: Precision agriculture, weather monitoring, and machine/process control.

### Chapter 2: Standards

This chapter maps the traditional Internet protocol stack to the highly constrained IoT environment.

#### 2.1 "Traditional" Internet Review

- Protocol Stack Model: Compares the theoretical 7-layer ISO-OSI model with the practical 4-layer TCP/IP model (Figure 2.1).

- Physical/Link Layers: IEEE 802.3 (Ethernet) for wired LANs; IEEE 802.11 (Wi-Fi, e.g., 802.11ac, 802.11ah, 802.11ai) using CSMA/CA for wireless LANs.

- Network Layer: IPv4 (32-bit addresses, ~4.3 billion limit) vs. IPv6 (128-bit addresses, default 1280-byte MTU). Figure 2.2 compares their headers.

- Transport Layer: TCP (connection-oriented, reliable, heavy 20-byte header) vs. UDP (connectionless, best-effort, lightweight 8-byte header).

- Application Layer: HTTP (Hypertext Transfer Protocol, text-based request/response), AMQP (Advanced Message Queuing Protocol), and SIP (Session Initiation Protocol).

#### 2.2 The Internet of Things

Translating the Internet stack to constrained smart objects requires low-power protocols.

- Physical/Link Layer:

  - IEEE 802.15.4: Low-rate wireless personal area networks (LR-WPAN). Imposes a strict maximum frame size of 127 bytes.

  - Bluetooth Low Energy (BLE): Uses generic attribute and access profiles for efficient sensor data collection.

- Network Layer (6LoWPAN): IPv6 over Low-power WPAN acts as an adaptation layer. Purpose: Compresses IPv6 headers and fragments IPv6 datagrams to fit within the 127-byte IEEE 802.15.4 MTU, and handles reassembly.

- Application Layer (CoAP - Constrained Application Protocol): Defined in RFC 7252.

  - Concept: A lightweight, binary, UDP-based RESTful protocol designed to easily map to HTTP (Figure 2.5).

  - Mechanisms: Uses a 16-bit Message-ID for duplicate detection/reliability and an 8-bit Token for asynchronous request/response matching. Supports 4 message types: CON (Confirmable), NON (Non-confirmable), ACK (Acknowledgement), and RST (Reset) (Figures 2.7-2.11).

  - Observe Option (RFC 7641): Replaces inefficient polling. Clients register via a GET request (Observe: 0); the server asynchronously pushes notifications when the resource state changes (Figures 2.15-2.16).

  - Block-Wise Transfers (RFC 7959): Solves UDP/MTU limits by splitting large payloads into manageable blocks (e.g., 128 bytes) using Block1 (requests) and Block2 (responses) options (Figures 2.17-2.19).

  - Cross-Proxies: Middleboxes translating HTTP to CoAP (H2C) to allow standard web browsers to communicate with IoT networks (Figures 2.22-2.24, Table 2.3 code mappings).

- Application Layer (CoSIP): A lightweight Constrained Session Initiation Protocol.

  - Purpose: Establishes, modifies, and tears down data sessions without carrying heavy CoAP headers in every subsequent data packet. Maps SIP semantics onto CoAP options (Figure 2.26).

### Chapter 3: Interoperability

This chapter focuses on architectural paradigms that ensure smart objects can seamlessly exchange data and evolve independently.

#### 3.1 & 3.2 The Verticals vs. The Web of Things

- The Problem: Traditional "Device-to-Cloud" silos fail in five areas: Scalability, Availability (fails if internet drops), Interoperability, Security, and System Evolution.

#### 3.3 REST Architectures (REpresentational State Transfer)

- Concept: An architectural style (used by HTTP and CoAP) promoting loose coupling between clients and servers. Resources are manipulated via representations and identified by URIs.

- HATEOAS: Hypermedia As The Engine Of Application State. The server guides the client through the application's finite-state machine (FSM) entirely via embedded links, ensuring the client requires no hardcoded knowledge of the server's backend (Figure 3.4).

- Richardson Maturity Model (RMM): Classifies REST compliance (Figure 3.5):

  - Level 0: Plain Old XML (POX) tunneling/RPC.

  - Level 1: Distinct URIs for distinct Resources.

  - Level 2: Proper use of HTTP Verbs (GET, POST, PUT, DELETE) and status codes (2xx, 4xx, 5xx) (Tables 3.1-3.3).

  - Level 3: Hypermedia (HATEOAS).

#### 3.5 Messaging Queues & Publish/Subscribe

- Concept: An asynchronous architecture (e.g., MQTT protocol) replacing client/server. Publishers push data to a central "Broker" organized by hierarchical topics. Subscribers register to topics. Advantage: Highly scalable, fully decouples endpoints (Figure 3.12).

#### 3.8 Dual-network Management Protocol (DNMP)

- Concept: A cross-layer management protocol combining a Low-Power/Low-Throughput (LPLT) radio (IEEE 802.15.4) and a High-Power/High-Throughput (HPHT) radio (IEEE 802.11s) on a single device.

- Operation: The LPLT network operates continuously. When a high-bandwidth task is needed (e.g., video streaming), a control message is sent over LPLT to selectively wake up ("Lock") the HPHT interfaces strictly along the required route, then turns them off ("Unlock") when finished, saving battery (Figures 3.19-3.21).

#### 3.9 & 3.10 Discoverability Data Formats (CoRE Link & SenML)

- CoRE Link Format (RFC 6690): standardizes discovery. Clients query /.well-known/core to receive a list of URIs and attributes (rt for resource type, if for interface type).

- SenML (Sensor Markup Language): Lightweight JSON/XML arrays formatting sensor data using standard labels (n=name, v=value, u=unit, t=time) (Table 3.9).

### Chapter 4: Discoverability

This chapter addresses how IoT devices automatically find services and resources without manual human configuration.

#### 4.1 & 4.2 Local and Large-scale Service Discovery

- Local: ZeroConf (mDNS/DNS-SD) allows devices to self-assign IPs and advertise services locally via multicast.

- Large-Scale: Relies on Peer-to-Peer (P2P) structured overlays (Distributed Hash Tables - DHT).

#### 4.3 Scalable Architecture for Service Discovery

- IoT Gateway: A border router functioning as a proxy, cache, and local Resource Directory (RD) (Figures 4.2-4.3).

- Distributed Location Service (DLS): A DHT acting as the "White Pages". Maps a specific URI to the contact address of the gateway managing that resource.

- Distributed Geographic Table (DGT): A spatial overlay acting as the "Yellow Pages". Answers geographic queries: DGT.get(lat, lon, rad) to find gateways managing resources within a specific radius (Figure 4.6).

- Performance: The operations logarithmically scale with network size, proving mathematical efficiency for billions of devices (Figure 4.12).

#### 4.4 Lightweight Service Discovery

- Local Filtered Flooding: Standard multicast flooding causes routing loops. Solved by integrating a Bloom Filter (a memory-efficient probabilistic data structure using add(x) and query(x) primitives) to record seen packets and drop duplicates (Figures 4.13-4.14).

- Multiple Unicast: Replaces physical layer broadcast with iterative unicast to synchronize duty-cycled nodes (ContikiMAC), drastically reducing latency and energy consumption (Figure 4.17-4.18).

### Chapter 5: Security and Privacy in the IoT

This chapter tackles cryptographic limitations and privacy enforcement in constrained environments.

#### 5.1 & 5.2 Security Mechanisms & Lightweight Cryptography (LWC)

- Goal: Ensure Confidentiality, Availability, Integrity, and Authenticity against cloning and DoS. Standard ciphers (AES/RSA) demand too much memory and power for Class 0/1 devices.

- Symmetric-key LWC:

  - TEA (Tiny Encryption Algorithm): High iteration, simple shifts/XORs, no preset tables (small footprint).

  - SEA (Scalable Encryption Algorithm): On-the-fly key derivation.

  - PRESENT: 64-bit block substitution-permutation network, highly efficient hardware implementation.

  - HIGHT: 64-bit Feistel network.

- Asymmetric-key LWC:

  - RSA: Based on large integer factorization (requires heavy 1024+ bit keys). Unsuited for IoT.

  - ECC (Elliptic Curve Cryptography): Based on algebraic points on curves. Highly suited for IoT (a 160-bit ECC key equals the security of a 1024-bit RSA key).

- Hash Functions: Keccak (SHA-3 family based on sponge functions), SPONGENT.

- Homomorphic Encryption: Allows intermediate aggregators to perform mathematical operations (like sums) directly on ciphertexts without decrypting them first: Em1⋅Em2=Em1+m2. Secures in-network data aggregation natively.

#### 5.3 Privacy Issues and IoT-OAS

- OAuth 2.0: Standard authorization defining roles: User (U), Service Provider (SP), Service Consumer (SC), and Authentication Service (AS).

- IoT-OAS (Open Authorization Service): Smart objects cannot run complex OAuth logic. IoT-OAS delegates authorization to the cloud. When a client requests data from a smart object, the object asks IoT-OAS to verify the client's token. IoT-OAS replies with an allow/deny grant (Figure 5.7).

### Chapter 6: Cloud and Fog Computing for the IoT

This chapter redesigns cloud analytics to accommodate the real-time demands of IoT data.

#### 6.1 & 6.2 Cloud Computing & Big Data Limitations

- Service Models: IaaS (Infrastructure), PaaS (Platform), SaaS (Software).

- Big Data Pipeline: Relies on batch processing. High latency occurs because the Data Warehouse (DW) must be actively polled by the consumer.

  - Latency Derivation: T=t0+t1+t2 (where t2 represents the wait/polling time of the consumer) (Figure 6.3).

#### 6.3 Big Stream Architecture

- Concept: Shifts from "process-oriented" (Big Data) to "consumer-oriented" (Big Stream). Data is dynamically pushed to registered "Listeners" via a directed graph architecture, eliminating polling.

  - Latency Derivation: T=t0+t1 (t2 is eliminated) (Figure 6.4).

- Graph-based Processing: Comprises a Core Graph (atomic operations: translations, normalizations) and an Application Graph (custom listener logic). Nodes can be dynamically replicated or turned off based on data flow load (Figures 6.5-6.7).

- Implementation Modules: Acquisition (NGINX/RabbitMQ), Normalization (JSON generation), Graph Framework, and Application Register (NRQM handles graph state).

#### 6.4 Big Stream and Security

- Concept: Protects Graph operations using the OFS (Outdoor Front-end Security) module for external streams and IGS (In-Graph Security) for internal isolation (Figures 6.16-6.17).

- Access Control: The PMSV (Policy Manager) and GRAN (Graph Rule Auth Notifier) enforce stream subscriptions matching rules. Processing nodes utilize in-node AES/RSA or homomorphic encryption to isolate their generated streams from unauthorized listeners (Figure 6.19).

#### 6.5 & 6.6 Fog Computing and the IoT Hub

- Fog Computing: Extends the cloud to the network edge, providing mobility support, high geographic distribution, and low-latency local processing.

- The IoT Hub: A pivotal fog node acting simultaneously as a Border Router, CoAP/HTTP Cross-Proxy, Resource Directory, and Cache (Figure 6.24).

- Virtualization & Replication: The cloud hosts virtual clones of the IoT Hub. An MQTT-based synchronization protocol ensures replicas are updated when resources are Polled, Observed, or Pushed (Figures 6.25-6.28). Replicas automatically scale based on consumer request rates, providing load balancing and extreme fault tolerance.

Here is the comprehensive and technically accurate analysis and summary of Chapter 7: The IoT in Practice, completing the book’s structured breakdown.

### Chapter 7: The IoT in Practice

This chapter bridges theoretical architectures with real-world deployments, detailing the hardware, software, testbed integrations, and user interaction patterns required to implement the Web of Things (WoT).

#### 7.1 Hardware for the IoT

To deploy the IoT, smart objects rely on specific hardware modules to sense, compute, and communicate.

- Hardware Architecture: A typical smart object comprises four modules:

  - Communication module: Radio transceiver (with antenna) or wired interface.

  - Microcontroller (MCU): The "brain" of the device, equipped with ROM (for storing program code) and RAM (for temporary data/packet buffers).

  - Sensors/Actuators: Interfaces to interact with the physical world.

  - Power Source: Usually batteries, but can include piezoelectric or solar harvesters.

7.1.1 Classes of Constrained Devices Following the RFC 7228 standard, IoT devices are strictly categorized by memory limits:

- Class 0 (C0): ≪10 KiB RAM and ≪100 KiB Flash. Too constrained to communicate securely with the Internet directly. They rely on proxies/gateways, are usually preconfigured, and act as basic health/status indicators.

- Class 1 (C1): ∼10 KiB RAM and ∼100 KiB Flash. Cannot run traditional stacks (HTTP/TCP/TLS) but can natively run lightweight protocols like CoAP over UDP.

- Class 2 (C2): ∼50 KiB RAM and ∼250 KiB Flash. Capable of running full traditional protocol stacks but still benefit from lightweight protocols to save bandwidth and energy.

7.1.2 Hardware Platforms Specific, market-available hardware platforms illustrate these device classes:

- TelosB: A Class 0 mote by UC Berkeley. Features a TI MSP430F1611 MCU, 10 kB RAM, 48 kB ROM, and an IEEE 802.15.4 transceiver (2400–2483.5 MHz). It is highly power-efficient (1.8 mA active, 5.1 μA sleep) and includes embedded light, temperature, and humidity sensors.

- Zolertia Z1: A Class 0 mote. Uses an MSP430F2617 MCU (8 kB RAM, 92 kB ROM), a CC2420 IEEE 802.15.4 transceiver, and features built-in 3-axis accelerometers and digital/analog bus connectivity (I2C, SPI, UART).

- OpenMote: A Class 0 modular hardware platform comprising three boards: OpenMote-CC2538 (the mote), OpenBase (programming/debugging), and OpenBattery (energy and sensor interfacing). It runs multiple OSs including Contiki, OpenWSN, FreeRTOS, and RIOT.

- Arduino Yún: A Class 2 development board featuring a dual-environment. It integrates an ATmega32U4 microcontroller (for real-time I/O) and an Atheros AR9331 Linux microprocessor (handling Wi-Fi/Ethernet connectivity).

- Raspberry Pi: A Class 2 Single Board Computer (SBC). The original Model B uses a Broadcom BCM2835 SoC (700 MHz ARM1176JZF-S) with 512 MB RAM. Newer iterations (Pi 3/Zero W) feature integrated 802.11n Wi-Fi and Bluetooth 4.1.

#### 7.2 Software for the IoT

Operating Systems (OS) for the IoT must be tailored to the strict RAM/ROM limitations of constrained devices while providing standard networking stacks.

- 7.2.1 OpenWSN: An open-source, standards-based stack running on top of the IEEE 802.15.4e Time-Slotted Channel-Hopping (TSCH) MAC layer. Coupled with 6LoWPAN, RPL, and CoAP, it enables ultra-reliable mesh networks and is highly hardware-independent.

- 7.2.2 TinyOS: An open-source, BSD-licensed OS designed for low-power WSNs. It is written in nesC (a C dialect optimized for concurrency) and uses a component-based, event-driven programming model.

- 7.2.3 FreeRTOS: A small, real-time operating system (RTOS) kernel. Mostly written in standard C to remain highly portable, it supports multiple threads, mutexes, semaphores, and software timers.

- 7.2.4 TI-RTOS: Texas Instruments' RTOS scales from a small preemptive multitasking kernel to a full suite equipped with power managers, TCP/IP, and USB stacks.

- 7.2.5 RIOT: A microkernel OS offering full multi-threading and real-time capabilities. It supports standard C/C++ and provides a native port to run directly as a process on Linux or macOS for simplified debugging.

7.2.6 Contiki OS Contiki is a premier open-source OS for memory-constrained IoT systems.

- Networking Stack: It provides the smallest certified uIPv6 stack, complete with 6LoWPAN header compression and RPL routing. It also offers the Rime stack, an alternative lightweight set of communication primitives for when IPv6 overhead is too high.

- Cooja Simulator: A powerful built-in network simulator that tests code on emulated hardware, native Cooja nodes, or Java nodes prior to physical deployment.

- Protothreads: The core programming abstraction of Contiki. It blends multi-threading and event-driven programming. Protothreads are cooperatively scheduled, meaning processes explicitly yield control to the kernel, drastically minimizing RAM overhead.

#### 7.3 Vision and Architecture of a Testbed for the Web of Things

The WoT Testbed (WoTT) aims to hide low-level implementation details, ensure self-configuration, and bridge multiple protocols/platforms into a seamless environment.

- 7.3.1 All-IP Infrastructure: The testbed groups nodes into Constrained IoT (CIoT, Class 1 using Contiki) and SBC nodes (Class 2 using Linux). CIoT nodes use IEEE 802.15.4 and 6LoWPAN/RPL, running the lightweight Erbium CoAP server. SBC nodes handle CoAP via Node.js, Python, or Java.

- 7.3.2 The IoT Hub Integration: The architecture centers around the IoT Hub, an edge-computing gateway (acting as an Origin Server, C2C Proxy, H2C cross-proxy, Resource Directory, and Cache). It translates external HTTP requests to internal CoAP interactions, merging heterogeneous link layers (802.11, 802.15.4) into a single IP network.

- 7.3.4 Wearable Applications: The WoTT validates human-object interactions. Wearable applications automatically discover resources (using ZeroConf and .well-known/core queries) and dynamically generate user interfaces based on the discovered capabilities.

#### 7.4 Wearable Computing for the IoT: Interaction Patterns in RESTful Environments

Bridging the gap between the mobile/wearable world and the physical IoT world requires standardizing interaction patterns to overcome fragmented, vendor-specific ("vertical") solutions.

- Interaction Flow: Humans interact with smart objects using Active Wearables (smartwatches) bridged via a smartphone. The sequence involves:

  - Discover: The smartphone proactively discovers nearby objects via ZeroConf or BLE beacons and queries the Resource Directory.

  - Select: The wearable interface displays available objects. The user selects one to view its hypermedia-driven function set.

  - Interact: The user operates the object via CoAP methods: Get (read state), Observe (subscribe to asynchronous updates), or Update (POST/PUT to actuate).

- Implementation: Evaluated on an Android Wear/Lollipop setup using Bluetooth (Wearable Data Layer API) to bridge the watch and phone, and CoAP (Californium library) to bridge the phone to 70 deployed smart objects.

#### 7.5 Effective Authorization for the Web of Things

While DTLS (CoAPs) secures transport, and the Delegated CoAP Authentication and Authorization Framework (DCAF) delegates DTLS channel establishment, they do not inherently solve third-party authorization. The IoT requires solutions for owner-to-owner and owner-to-any authorization profiles.

7.5.1 Authorization Framework Architecture This framework extends the IoT-OAS (Internet of Things - Open Authorization Service) using OAuth 2.0 to grant Access Tokens (AT) holding specific permission tuples: ⟨res,act,exp⟩ (Resource URI, REST action, Expiration time).

- Owner-to-Owner: A user (u1) scans an object's UUID (e.g., via QR code). u1 sends a register(UUID) API call to IoT-OAS. IoT-OAS binds the device to u1, blocks further registrations, and grants an "all-access" token to u1.

- Reactive Owner-to-Any: A secondary user (u2) discovers u1's object. u2 sends request(UUID, P) to IoT-OAS, which forwards it to the owner (u1). u1 approves it by sending grant(u2, UUID, Pg) to IoT-OAS. IoT-OAS then appends the granted permissions to u2's access token.

- Proactive Owner-to-Any: The owner (u1) directly issues a grant(u2, UUID, P) message to IoT-OAS, pre-authorizing u2. u2 is notified via push mechanisms.

- Service Provider Verification: In all cases, when u2 attempts to interact with the smart object, the object (Service Provider) simply delegates the security check by calling verify(request) on the IoT-OAS, stripping complex access-control logic out of the constrained device.

7.5.2 Implementation and Validation The authorization framework was fully implemented utilizing CoAP/CoAPs with the Californium library and its Scandium DTLS 1.2 module. Users manage grants through an Android application utilizing external authentication (Facebook/Google+) to log into the IoT-OAS system, seamlessly enabling fine-grained, secure sharing of physical devices in environments like office laboratories.