# Multimedia

# Part I: Introduction and Multimedia Data Representations

## Chapter 1: Introduction to Multimedia

### 1.1 What is Multimedia?

- Multimedia Definition: Applications utilizing multiple modalities (text, images, graphics, animation, video, sound, speech, and interactivity) to enhance communication.

- Research Topics: The field spans multimedia processing and coding (compression, content analysis), system support and networking (protocols, operating systems), and tools/end systems (interfaces, hypermedia).

### 1.2 Multimedia: Past and Present

- Early History: The concept of nonlinear associative memory was introduced by Vannevar Bush's "Memex" in 1945. Douglas Engelbart's NLS (On-Line System, 1968) introduced mouse pointing and hypertext links. Thomas Edison invented the phonograph (analog sound reproduction) in 1877 (Figure 1.2). Early imaging includes the dry-plate camera (Figure 1.1).

- Hypertext and Hypermedia: Ted Nelson coined hypertext to describe nonlinear text reading (Figure 1.4). Tim Berners-Lee proposed the World Wide Web in 1990 using HTML (HyperText Markup Language), which utilizes structural tags.

- XML (Extensible Markup Language): Solves HTML's structural rigidity by modularizing data, using a Document Type Definition (DTD) for structure and XML Style Sheets (XSL) for rendering.

### 1.3 Multimedia Software Tools: A Quick Scan

- Audio/Music: Sequencing tools (Cakewalk, Finale, Sibelius) handle MIDI and notation; Digital Audio tools (Pro Tools) manipulate waveforms.

- Graphics and Image Editing: Adobe Illustrator handles vector graphics; Adobe Photoshop manipulates pixel-based images using tools like the alpha channel and magic wand.

- Video Editing and Animation: Tools like Adobe Premiere and CyberLink PowerDirector provide nonlinear video composition via tracks. Adobe Flash generates 2D web animation using symbol libraries, tweening, and ActionScript.

- Multimedia Authoring: Adobe Director uses a movie metaphor (Cast, Stage, Score, and Lingo scripts). Dreamweaver allows HTML-free web authoring.

### 1.4 & 1.5 Multimedia in the Future and Exercises

- Executable Academic Papers: A future application where embedded data and code can be interactively manipulated by the reader.

## Chapter 2: A Taste of Multimedia

### 2.1 & 2.2 Multimedia Tasks and Presentation

- Computer Vision Hierarchy: Extends from low-level image segmentation up to object detection and high-level scene recognition.

- Presentation Guidelines: Sans serif fonts are optimally readable for projections (Figure 2.1). Human retinas cannot simultaneously focus on highly contrasting adjacent colors (e.g., red and blue).

- Color Contrast Principle: Text legibility is maximized when the background is the principal complementary color (Figure 2.2).

  - Equation 2.1: R,G,B→1−R,1−G,1−B.

- Sprite Animation: Achieved by compositing a sprite (S) over a background (B) using a binary mask (M) via logical operations (ANDing B and M, ORing with S) (Figure 2.5).

- Video Transitions: Used to signal semantic scene changes (Figures 2.6, 2.7).

  - Type I Cross Dissolve (Equation 2.2): D=1−αt⋅A+αt⋅B.

  - Transition Function (Equation 2.3): αt=kt, constrained by ktmax≡1.

  - Slide Transition: Translations mapping pixels from fixed source images to shifting viewports (Figure 2.8, pseudocode). Other transitions include the Iris wipe (Figure 2.22) and Clock wipe (Figure 2.23).

### 2.3 Data Compression

- Necessity: Standard-definition uncompressed video requires ~28 MB/s, while HD video requires ~187 MB/s, necessitating compression (Table 2.1).

- JPEG Quality Factor (Q): Dictates the lossy decimation level (0-100). Decreasing Q scales file size down algorithmically but introduces severe blocky artifacts below Q=25 (Table 2.2, Figure 2.9).

### 2.4 to 2.7 Production, Distribution, and Tools

- Workflow: Includes building storyboards (keyframe sketches) and flowcharts (interactive navigation hierarchies).

- Social Media Distribution: Platforms like YouTube utilize user tags and Content Distribution Networks (CDNs) to manage petabyte-scale media delivery (Figure 2.10, 2.11).

- Editing Tools: Adobe Premiere uses a nonlinear score timeline (Figure 2.13, 2.14); Flash uses tweening (Figure 2.18). Figure 2.24 demonstrates spatial pixel-offset transformations.

## Chapter 3: Graphics and Image Data Representations

### 3.1 Graphics/Image Data Types

- 1-Bit Images: Binary monochrome representation (Figure 3.1).

- 8-Bit Gray-Level Images: Contains 256 intensity states (Figure 3.3).

  - Dithering: Approximates halftones for 1-bit printers using n×n dot patterns (Figure 3.4).

  - Ordered Dither Algorithm (Alg 3.1): Projects intensities into a dither matrix D. Ox,y=1 if Ix,y>Di,j else 0; where i=x mod n and j=y mod n.

- 24-Bit Color Images: Uses 8 bits each for RGB, creating 16,777,216 colors (Figure 3.5).

  - Multispectral imaging extends this to many more bands for satellite data.

- 8-Bit Color Images: Maps colors via an index to a 256-color Color Lookup Table (LUT) (Figures 3.6, 3.7, 3.9).

  - Median-Cut Algorithm: Extracts an optimal palette by recursively splitting a 3D RGB color histogram (Figure 3.11) at the median of the longest dimension. Applied dithering masks gradient banding (Figure 3.10).

### 3.2 Popular File Formats

- File Format Matrix: Table 3.1 lists standard formats supported by editing tools.

- GIF (Graphics Interchange Format): Uses LZW lossless compression, limited to 8-bit color. Contains a Screen Descriptor (Figure 3.12), Color Map (Figure 3.14), and Image Descriptor (Figure 3.15). Features a 4-pass interlace display (Rows 0/8, 4/12, 2/6, 1/3) for progressive loading (Figure 3.16).

- JPEG: Optimizes for the human vision system by decimating high-spatial-frequency data in the frequency domain (Figure 3.17).

- Other Formats: PNG (lossless, alpha channels), TIFF, BMP, WMF, Netpbm (PNM/PAM), EXIF (camera metadata), and Postscript/PDF.

- PTM (Polynomial Texture Mapping): Interpolates surface lighting utilizing regression models over n camera images taken under varying light directions (Figure 3.18).

- Figure 3.19: Visualizes bitplane distribution calculations for grayscale ramps.

## Chapter 4: Color in Image and Video

### 4.1 Color Science

- Light and Spectra: Visible light spans 400-700 nm. Its Spectral Power Distribution (SPD) Eλ defines energy across wavelengths (Figures 4.1, 4.2).

- Human Vision: The retina utilizes rods (luminance) and cones (color receptors) qλ (Figure 4.3).

  - Cone Vector (Equation 4.1): qλ=qRλ,qGλ,qBλT.

  - Eye Response (Equations 4.2 & 4.3): Factoring in illuminant Eλ, object surface reflectance Sλ (Figure 4.4, 4.5), and cone sensitivity: R=∫EλSλqRλdλ.

- Gamma Correction: Pre-compensates for the nonlinear light emission (L∝Vγ) of Cathode Ray Tubes (CRTs) (Figures 4.6, 4.7).

  - Equation 4.4: R′=R1/γ→R′γ→R.

  - SMPTE 170M standard (Equation 4.5): Uses a piecewise linear and power-law transform to avoid infinite derivatives at zero voltage.

- CIE Color-Matching Functions: Defines strictly positive functions xλ,yλ,zλ (Figures 4.8, 4.9, 4.10) yielding Tristimulus values.

  - Equation 4.6: X=∫Eλxλdλ.

- CIE Chromaticity Diagram: A 2D locus projecting color independent of magnitude (Figure 4.11).

  - Equations 4.7, 4.8, 4.9: x=X/X+Y+Z, y=Y/X+Y+Z, z=1−x−y.

  - Standard Illuminants: D65, A, C define varied white points (Figure 4.12). Monitor gamuts limit printable/displayable colors (Figure 4.13, Figure 4.21, Table 4.1).

- White Point Correction & Transform Matrices:

  - Equation 4.10: Basic conversion X=xrR+xgG+xbB.

  - Equations 4.11 to 4.14: Calculates a diagonal correction matrix D=diagd1,d2,d3 ensuring Ywhite≡1 yielding XYZwhite≡MD1,1,1T.

  - Equations 4.15 to 4.18: Establishes transform matrix T=MD to definitively map RGB to XYZ.

- CIELAB (L*a*b*) Color Model: A perceptually uniform model using cube-root power laws mimicking Weber's Law (Figure 4.14).

  - Color Distance (Equation 4.21): ΔE=L1*−L2*2+a1*−a2*2+b1*−b2*2.

  - Equations 4.22 & 4.23: Defines luminance L*, red-green a*, yellow-blue b*, chroma, and hue angle.

### 4.2 & 4.3 Color Models in Images and Video

- Camera-Dependent Color: Direct transformations map standard coordinates to specific camera sensor sensitivities (Equations 4.24-4.26).

- Subtractive Color (CMY/CMYK): Used for printers (Figures 4.15, 4.16, 4.17).

  - Equations 4.27 & 4.28: C=1−R, M=1−G, Y=1−B.

  - Undercolor Removal (Equation 4.29): Extracts true black (K=minC,M,Y) to prevent muddy color mixing.

- YUV: Separates luminance (Y′) and chrominance (U,V) for analog PAL video (Figure 4.18).

  - Equation 4.19 & 4.20: Luminance Y′=0.299R′+0.587G′+0.114B′.

  - Equations 4.30 to 4.35: Maps combinations to maintain analog composite tolerances (C=Ucosωt+Vsinωt).

- YIQ: Modifies YUV for NTSC via a 33-degree rotation (Figure 4.19).

  - Equations 4.36 to 4.38: I=0.492R′−Y′cos33∘−0.877B′−Y′sin33∘.

- YCbCr: Scaled digital equivalent optimized for quantization.

  - Equations 4.39 to 4.42: Dynamically centers chrominance Cb=B′−Y′/1.772+0.5, clamped for digital studio standards. Figure 4.20 illustrates mathematical mappings of matching functions.

## Chapter 5: Fundamental Concepts in Video

### 5.1 & 5.2 Analog and Digital Video

- Interlaced Scanning: Scans odd then even fields to reduce flicker without doubling bandwidth (Figure 5.1, Figure 5.2). Includes horizontal/vertical retrace blanks containing sync data (Figure 5.4).

- Analog Formats: Table 5.1 details NTSC (525 lines, 29.97 fps) intertwining luma and chroma spectra (Figure 5.5). Table 5.2 details PAL/SECAM (625 lines, 25 fps).

- Digital Chroma Subsampling: Exploits human insensitivity to color detail (Figure 5.6). Subsampling schemes include 4:4:4 (none), 4:2:2 (horizontal), 4:1:1, and 4:2:0 (horizontal/vertical decimation).

- Digital Standards: Table 5.3 defines CIF and QCIF resolutions. Table 5.4 details HDTV and UHDTV parameters (progressive scanning up to 8K resolution).

### 5.3 & 5.4 Interfaces and 3D Video

- Interfaces: Analog includes Component/Composite/S-Video/VGA (Figure 5.7); Digital includes DVI/HDMI/DisplayPort (packetized transmission).

- Stereopsis and Disparity: Horizontal parallax triggers depth perception. Simple vs Toed-in camera models generate offset views.

  - Equation 5.3: Disparity d=fb/Z. Disparity is inversely proportional to depth.

- Vergence-Accommodation Conflict: A perceptual flaw in 3D displays where the eyes focus (accommodate) on the fixed screen plane but converge on simulated 3D depths, inducing fatigue (Figure 5.9).

- Autostereoscopic Devices: Lenticular lenses and parallax barriers negate the need for 3D glasses.

## Chapter 6: Basics of Digital Audio

### 6.1 Digitization of Sound

- Acoustics: Sound is a continuous 1D longitudinal pressure wave (Figure 6.1, 6.2).

- Nyquist Theorem: Sampling quantizes time (Figure 6.3). The sampling rate must strictly exceed 2fmax to prevent aliasing (false lower-frequency artifact reconstruction) (Figure 6.4, 6.5).

- SNR and SQNR: Defines quality (Table 6.1).

  - Equation 6.2: SNR=10log10Vsignal2/Vnoise2.

  - Equation 6.4: For N-bit quantization, SQNR≈6.02N+1.76 dB. (Derived via RMS of quantization error).

- Nonlinear Quantization (Companding): Adjusts dynamic ranges using logarithms based on Weber's Law, assigning finer granularity to quiet sounds (Figure 6.6, 6.7).

  - Equations 6.5 to 6.8: Weber's law derivation r=klns/s0.

  - Equation 6.9 (μ-law): r=signsln1+μs/spln1+μ.

  - Equation 6.10 (A-law) & 6.11 (Decoder): Piecewise variants and inverse mapping.

- Audio Filtering & Synthesis: Quality correlates linearly to bandwidth (Table 6.2).

  - Equation 6.12: Complex FM synthesis modeling overtones xt=Atcosωcπt+Itcosωmπt+ϕm+ϕc (Figure 6.8).

### 6.2 MIDI (Musical Instrument Digital Interface)

- Architecture: Encodes event commands (Note On, Velocity) instead of waveforms, saving space (Figure 6.9, 6.10, 6.11, 6.12).

- Messages: Utilizes Status Bytes (MSB=1) and Data Bytes. Employs "Running Status" to omit redundant bytes. Categories include Voice, Mode, and System messages (Figure 6.13, Tables 6.3, 6.4, 6.5).

### 6.3 Quantization and Transmission

- PCM (Pulse Code Modulation): Translates amplitudes into fixed uniform digital pulses (Figure 6.14, 6.15).

- Lossless Predictive Coding: Transmits the difference between the true signal fn and predicted signal fn, reducing signal variance drastically (Figure 6.16, 6.17).

  - Equation 6.13: Error en=fn−fn.

  - Equation 6.14: Linear predictor fn=∑an−kfn−k.

  - Equations 6.15, 6.16: Example predictive truncation algorithms.

- DPCM (Differential PCM): A lossy form appending a quantizer to predictive coding (Figure 6.18, Table 6.7).

  - Equation 6.17: en=Qen.

  - Equations 6.18, 6.19: Lloyd-Max thresholding to minimize distortion error.

  - Equations 6.20, 6.21: Truncated quantization tracking derivations.

- DM (Delta Modulation): Simplifies difference coding to a 1-bit (-k or +k) slope tracker.

  - Equation 6.22: Uniform DM. Adaptive DM scales k dynamically based on curve severity.

- ADPCM (Adaptive DPCM): Dynamically scales predictive steps using Wiener-Hopf equation systems (Figure 6.19, 6.20).

# Part II: Multimedia Data Compression

## Chapter 7: Lossless Compression Algorithms

### 7.2 Basics of Information Theory

- Entropy: Defined by Shannon, entropy is the theoretical minimum average bits required per symbol in an information source.

- Entropy Equation: η=−∑pilog2pi. The average codeword length l is strictly bounded by η≤l.

### 7.4 Variable-Length Coding (VLC)

- Shannon-Fano Algorithm: A top-down approach that sorts symbols by frequency and recursively splits them into two halves of approximately equal probabilities, assigning 0s to left branches and 1s to right branches.

- Huffman Coding: A bottom-up, optimal prefix code generation algorithm. It iteratively combines the two lowest-frequency nodes into a parent node.

- Extended Huffman Coding: Groups k symbols together to bring the average bitrate closer to the entropy bound: η≤l<η+1/k.

- Adaptive Huffman Coding: Dynamically updates the tree without prior statistics. It maintains the "sibling property" by swapping nodes whenever a count update violates the frequency-sorted tree order.

### 7.5 & 7.6 Dictionary-Based and Arithmetic Coding

- LZW Compression: Emits fixed-length codes for variable-length strings by adaptively building a dictionary. It maps concatenations of known characters to new codes seamlessly.

- Arithmetic Coding: Treats the entire message sequence as a single fractional interval [low,high). The interval shrinks based on symbol probabilities.

- Scaling (E1, E2, E3): Overcomes floating-point underflow. E1 and E2 scale intervals falling strictly in the top or bottom halves; E3 scales intervals straddling the midpoint.

### 7.7 Lossless Image Compression

- Lossless JPEG: Uses spatial differential prediction instead of the DCT. Predictors linearly combine up to three causal neighboring pixels (e.g., P4=A+B−C), and the residuals are entropy-coded.

## Chapter 8: Lossy Compression Algorithms

### 8.2 & 8.4 Distortion and Quantization

- Distortion Measures: Mean Square Error (MSE) is σd2=1N∑xn−yn2. Peak Signal-to-Noise Ratio (PSNR) measures quality logarithmically.

- Uniform Quantization: Divides the domain into equal intervals. Midrise quantizers lack a zero level; midtread quantizers feature a zero level (dead-zone).

- Lloyd-Max Quantizer: Iteratively minimizes MSE for non-uniform sources by recalculating boundaries bj=yj+1+yj/2 and centroids yj=∫xfXxdx/∫fXxdx.

- Vector Quantization: Maps input vectors to the closest code vector within a shared codebook, outputting just an index.

### 8.5 Transform Coding (DCT & KLT)

- 1D and 2D Discrete Cosine Transform (DCT): Decorrelates spatial data into spatial frequencies.

  - 1D DCT: Fu=Cu2∑cos2i+1uπ16fi.

  - 2D Matrix Implementation: Factorized into two 1D transforms via orthogonal matrix T: F=T⋅f⋅TT.

- Karhunen-Loève Transform (KLT): The theoretically optimal, data-dependent transform. Diagonalizes the autocorrelation matrix RX=EXXT via eigenvectors.

### 8.6 & 8.8 Wavelet-Based Coding and EZW

- Discrete Wavelet Transform (DWT): Provides both time and frequency localization. Filters signals into low-pass and high-pass components, yielding LL, HL, LH, and HH subbands.

- Embedded Zerotree Wavelet (EZW): Takes advantage of cross-scale self-similarity. If a parent coefficient is smaller than a threshold, all spatially corresponding descendants are coded as a single "Zerotree Root" symbol. Applies Successive Approximation Quantization (SAQ) bitplanes.

- SPIHT: Set Partitioning in Hierarchical Trees improves EZW by partitioning subsets of coefficients recursively.

## Chapter 9: Image Compression Standards

### 9.1 The JPEG Standard

- Workflow: RGB to YCbCr → Subsampling → 8x8 DCT → Quantization → Zigzag Scan → Entropy Coding.

- Quantization: High frequencies are attenuated using a user-scaled matrix Q. Fu,v=roundFu,v/Qu,v.

- Entropy Coding: DC coefficients are DPCM-coded against the previous block's DC. AC coefficients are scanned in a zigzag pattern to maximize zero-runs and are encoded via Run-Length Coding (RLC).

- Modes: Sequential (default), Progressive (spectral selection or successive approximation), Hierarchical (multiresolution pyramid), and Lossless.

### 9.2 JPEG2000

- EBCOT: Embedded Block Coding with Optimized Truncation. Uses the DWT, isolates coefficients into code blocks, and bitplane-codes them through forward-significance, reverse-significance, and magnitude refinement passes.

- Post-Compression Rate-Distortion (PCRD): Optimizes bitstreams by finding optimal truncation points for each code block to meet bitrate constraints.

- ROI Coding: Shifts Region of Interest coefficients up so they are prioritized during bitplane transmission.

### 9.3 & 9.4 JPEG-LS and JBIG

- JPEG-LS: Uses the LOCO-I context model to predict pixels directly via a causal spatial template.

- JBIG/JBIG2: Bi-level standards that use Dictionary-based pattern matching (substitution) to encode recurring text characters natively.

## Chapter 10: Basic Video Compression Techniques

### 10.2 & 10.3 Motion Compensation and Search Vectors

- Theory: Instead of storing full frames, blocks in the Target frame are subtracted from matched blocks in a Reference frame. The displacement is stored as a Motion Vector (MV).

- Search Algorithms: Aim to minimize Mean Absolute Difference (MAD).

  - Sequential Search: Exhaustive comparison of all pixels, computationally bounded by Op2N2.

  - 2D Logarithmic Search: Iteratively reduces the search window by half.

  - Hierarchical Search: Creates a downsampled pyramid, estimates coarse MVs at low resolutions, and refines them at higher resolutions.

### 10.4 & 10.5 H.261 and H.263

- H.261: Telephony standard supporting QCIF and CIF. Introduces I-frames (Intra, standalone) and P-frames (Inter, predictive). Employs a constant step_size quantizer equation: F=⌊DCT/2×scale⌋.

- H.263: Adds subpixel (half-pixel) precision via bilinear interpolation. Introduces PB-frames, coding a P-frame and a B-frame as one unit to save bandwidth.

## Chapter 11: MPEG Video Coding: MPEG-1, 2, 4, and 7

### 11.2 & 11.3 MPEG-1 and MPEG-2

- MPEG-1: Incorporates B-frames (Bidirectional prediction from past and future frames) to maximize compression. Organizes video into Slices (independent horizontal strips) for synchronization.

- MPEG-2: Targets digital broadcast TV (interlaced).

  - Interlaced Support: Encodes via Frame-pictures or Field-pictures with alternate zigzag scans.

  - Scalabilities: Supports SNR, Spatial, Temporal, and Hybrid scalabilities using interacting base and enhancement layers.

### 11.4 MPEG-4

- Object-Based Coding: Treats video content as independent Video Object Planes (VOPs).

- Binary Shape Coding: Encodes non-rectangular bounds using Binary Alpha Blocks (BABs) processed via Context-based Arithmetic Encoding (CAE).

- Global Motion Compensation: Warps perspective using 8-parameter affine matrices over panoramic Sprite backgrounds.

- Synthetic Objects: Represents 2D objects via Delaunay mesh structures and 3D objects with Face Animation Parameters (FAPs).

### 11.5 MPEG-7

- Multimedia Content Description Interface: Standardizes metadata using Descriptors (D), Description Schemes (DS), and a Description Definition Language (DDL) utilizing XML Schema.

## Chapter 12: New Video Coding Standards: H.264 and H.265

### 12.1 H.264 (AVC)

- Variable Block Sizes & Interpolation: Macroblocks partition down to 4x4. Interpolates quarter-pixel targets using a 6-tap FIR filter.

- Integer Transform: Replaces the standard DCT with a drift-free exact 16-bit Integer Transform: F=roundH⋅f⋅HT/215.

- In-Loop Deblocking Filter: Actively smooths internal block boundary edges immediately after inverse quantization based on threshold limits αQP and βQP.

- Entropy Coding: Utilizes CAVLC (Context-Adaptive Variable Length Coding) mapping variables to neighboring block counts, and CABAC (Context-Adaptive Binary Arithmetic Coding) tracking Markov probabilities contextually.

### 12.2 H.265 (HEVC)

- Quadtree Structure: Drops macroblocks in favor of Coding Tree Units (CTUs) up to 64x64, containing partitioned Coding Blocks, Prediction Blocks, and Transform Blocks.

- Parallel Processing: Incorporates Tiles (rectangular bounds) and Wavefront Parallel Processing (WPP) to thread coding rows.

- Transforms: Uses Discrete Sine Transform (DST) specifically for 4x4 intra-predicted blocks.

## Chapter 13: Basic Audio Compression Techniques

### 13.1 ADPCM

- Jayant Quantizer: A backward-adaptive quantizer that scales the step size strictly based on the previous output: Δ←MkΔ.

### 13.3 Vocoders

- Phase Insensitivity: Human hearing analyzes the magnitude spectrum, generally ignoring phase.

- LPC (Linear Predictive Coding): Predicts the vocal tract as a linear filter. Analyzed via Wiener-Hopf equations minimizing Esn−∑ajsn−j2 to extract prediction coefficients aj.

- CELP (Code Excited Linear Prediction): Uses an Analysis-by-Synthesis closed-loop approach. Matches Long Time Predictions (pitch) via an Adaptive Codebook and Short Time Predictions via a Stochastic Codebook.

- MELP: Improves LPC robustness in noisy environments by representing periodic excitation as a dispersed waveform.

## Chapter 14: MPEG Audio Compression

### 14.1 Psychoacoustics

- Threshold of Hearing: Identifies frequencies at which humans are deaf.

- Frequency Masking: Strong sounds raise the threshold of hearing for nearby frequencies. Modeled in "Barks" equating to the width of one critical band.

- Temporal Masking: Loud sounds desensitize auditory receptors slightly before and significantly after the sound event.

### 14.2 MPEG Audio

- Subband Bit Allocation: Routes signals through filter banks. Computes the Signal-to-Mask Ratio (SMR) and allocates bits sequentially to subbands with the lowest Mask-to-Noise Ratio (MNR).

- MP3 (Layer 3): Applies a Modified Discrete Cosine Transform (MDCT) over 50% overlapping windows to eradicate boundary artifacts. Uses a dual-nested loop for iterative bit/noise allocation.

- AAC (MPEG-4): Improves efficiency using Spectral Band Replication (SBR) in High Efficiency (HE-AAC) modes.

## Chapter 15: Network Services and Protocols

### 15.1 - 15.3 Network Layers & IP

- Models: OSI defines 7 layers; TCP/IP simplifies this. TCP provides window-based, connection-oriented delivery using AIMD flow control. UDP provides connectionless, best-effort datagram delivery suited for real-time video.

- Addressing & NAT: IPv4 provides 32-bit addresses, causing shortages mitigated by NAT (Network Address Translation) rewriting port headers. IPv6 extends this to 128 bits.

### 15.4 & 15.5 Multicast and QoS

- IP Multicast: Group dissemination managed by IGMP (Internet Group Management Protocol) queries.

- QoS (Quality of Service): DiffServ utilizes router-level queues (Expedited Forwarding for low-loss video). RSVP uses receiver-initiated soft-states to reserve path bandwidth dynamically.

### 15.6 Multimedia Protocols

- RTP (Real-Time Transport Protocol): Attaches timestamps and sequence numbers to UDP payloads.

- RTCP: Feeds QoS metrics back to the sender.

- RTSP: Offers out-of-band VCR control functions (PLAY, PAUSE).

- SIP: Session Initiation Protocol uses a text-based request framework via Proxy/Redirect servers.

## Chapter 16: Internet Multimedia Content Distribution

### 16.1 Proxy Caching

- Strategies: Caching reduces start-up latency. Methods include sliding-interval (temporary storage), prefix (first moments of video), segment (specific hotspots), and rate-split caching (stores VBR peaks to normalize backbone transmission to constant limits: Dt≤At≤Wt).

### 16.2 to 16.4 CDNs & Broadcast

- CDN: Akamai uses DNS routing to map clients to nearby Edge Servers, using Reflectors for path redundancy.

- Broadcast VoD: Protocols logically divide server bandwidth B into channels to minimize user wait time δ. Schemes include Staggered, Skyscraper, and Harmonic Broadcasting.

### 16.6 & 16.7 P2P and DASH

- Peer-to-Peer (CoolStreaming): Clients track local chunk availability via a 120-segment Buffer Map (BM) sliding window and pull segments dynamically.

- DASH: Dynamic Adaptive Streaming over HTTP fragments video representations. Clients use standard HTTP GET requests to fetch optimal resolution chunks based on local bandwidth.

## Chapter 17: Multimedia Over Wireless and Mobile Networks

### 17.1 & 17.2 Wireless Channels & Networking

- Fading: Modeled by Rayleigh and Rician distributions, multipath fading causes destructive signal interference.

- Cellular Modulations: Progressed from FDMA/TDMA (frequency/time slots) to CDMA (Spread Spectrum assigning unique orthogonal codes to users utilizing full bandwidth).

- WLAN (802.11): Uses CSMA/CA (Collision Avoidance) handling the "hidden terminal" problem.

### 17.3 Error Resilience

- FEC (Forward Error Correction): Appends redundancy data. Includes Block Codes (Hamming, Reed-Solomon for packet recovery) and Convolutional Codes (RCPC for Unequal Error Protection).

- Concealment: Uses Reversible Variable Length Codes (RVLCs) to read damaged entropy streams bidirectionally, preventing error propagation.

### 17.4 Mobility

- Mobile IP: Utilizes Home Agents (HA) and Foreign Agents (FA) to tunnel packets to moving nodes.

- Handoff: Hard handoff drops old connections before connecting; Soft handoff (CDMA) maintains concurrent connections temporarily.

## Chapter 18: Social Media Sharing

### 18.2 YouTube Characteristics

- Network Graphs: Media relations exhibit Small-World network qualities, characterized by a large clustering coefficient and a short random-graph characteristic path length.

### 18.3 Media Propagation

- Epidemiological Models: Spread dynamics are tracked using the SI2RP model (Susceptible - Immune - Infectious - Recovered - Permanent), dictating state transitions via probabilities of viewing and sharing pV and pS.

- COOLS: Coordinated Live Streaming incorporates latency-tolerant storage nodes tightly with low-latency streaming nodes by actively merging and promoting overlay trees.

## Chapter 19: Cloud Computing for Multimedia Services

### 19.1 Service Models

- Structure: Comprises Infrastructure as a Service (IaaS), Platform as a Service (PaaS), and Software as a Service (SaaS). Amazon AWS implements this via Elastic Compute Cloud (EC2) instances backed by S3 (Simple Storage Service) and EBS (Elastic Block Store).

### 19.4 Computation Offloading

- CAME: Cloud-Assisted Motion Estimation transfers heavy 2D mesh-based motion calculations off mobile devices into the cloud, yielding net energy savings for the local hardware.

### 19.5 Cloud Gaming

- Architecture (Gaikai): Captures game logic on powerful GPUs in the cloud and utilizes low-latency UDP streams to transmit rendered H.264 video down to thin clients, abstracting away local hardware limits.

## Chapter 20: Content-Based Retrieval in Digital Libraries

### 20.3 & 20.4 CBIR Features & Quantifying

- C-BIRD: Queries utilizing image-processing metrics such as Color Density, Color Layout, and Texture Layout (Gabor/edge gradients).

- Quantifying: Search reliability is represented via a Receiver Operating Characteristic (ROC) curve evaluating False Positives vs True Positives. Uses Mean Average Precision (MAP) to average recall points.

### 20.5 & 20.7 Key Technologies

- SIFT: Scale Invariant Feature Transform isolates robust local key points via a Difference of Gaussian (DOG) cascade filter across image octaves.

- Video Querying: Human Activity Modeling classifies spatio-temporal video data into actions, social roles, and events through structured Support Vector Machine (SVM) inference scoring.